| METROLOGY PROCEDURE: part b: Metering Data Validation, Substitution and Estimation Procedure for Metering Types 1 – 7 |  |
| --- | --- |
| PREPARED BY: | Market Development |
| DOCUMENT NO: | MT_MA1680 |
| VERSION NO: | 5.3x |
| EFFECTIVE DATE: | TBC |
| STATUS: | Draft for workshop discussion |

Preface

The metrology procedure has been prepared by the Australian Energy Market Operator (AEMO) in accordance with the requirements of the Rules.

This version of the Metrology Procedure: Part B has been updated to facilitate discussion on changes required as a consequence of the Metering Competition Rule change and is based on AEMO’s understanding og the draft determination.

The effective date of the metrology procedure is TBC, in accordance with the Rules.

Important Notice

This Metrology Procedure (Part B) is made by AEMO under clause 7.14 of the National Electricity Rules (Rules), and has effect only for the purposes set out in the Rules.  The Rules and the National Electricity Law prevail over this Procedure to the extent of any inconsistency.

© 2015 Australian Energy Market Operator.  The material in this publication may be used in accordance with the copyright permissions on AEMO’s website.

Document History

| Version | Date | Author | Comments |
| --- | --- | --- | --- |
| 1.00 | June 1997 | Multi – Jurisdictional Working Group | Original procedure – Nomenclature, Guidelines and procedures for National Metering Identifier, Registration and Transfer, Data Substitution, Data validation and Transmission Node Identifiers. |
|  | September 1997 | AEMO | Incorporated as part of the Service Level Requirements for Metering Data Agents. |
| 2.00 | August 2000 | MSWG | NEM Data Substitution and Validation Procedure published as a procedure. |
| 3.00 | August 2001 | MSWG | Procedure updated to include estimation and substitution for metering types 5, 6 and 7. |
| 4.00 | March 2008 | MSWG | Procedure updated to include clarifications and the process by which accumulated data and unmetered loads are converted into trading intervals. Procedure to become Part B of the metrology procedure. |
| 4.01 | April 2009 | AEMO | Uppdated to incorporate Victorian jurisdictional metrology material to support Victorian AMI rollout |
| 4.01a | 15 July 2011 | AEMO | The extension of retail contestability to business customers consuming between 50 – 150 MWh per annum in Tasmania. |
| 5 | 31 October 2011 | AEMO | Updated to incorporate provisions of Rules Change: Provision of Metering Data Services and Clarification of Existing Metrology Requirements. |
| 5.10 | 1 January 2014 | AEMO | Introduction of FRC in Tasmania |
| 5.20 | 1 March 2014 | AEMO | Updated to incorporate provisions of Rules Change: National Electricity Amendment (Small Generation Aggregator Framework) Rule 2012 ERC 0141 and text corrections |
| 5.30 | 15 May 2015 | AEMO | Updated to incorporate traffic signal dimming, introduce type 65 substitutions and text corrections. |
|  |  | AEMO | Updated to include MC rule change |

Table of Contents

1	General	8

1.1	Introduction	8

1.2	References	8

1.3	Definitions	8

1.4	General data validation, substitution and forward estimation requirements	8

1.5	Metering data substitution requirement	9

1.6	Metering data forward estimation requirement	10

1.7	Metering data quality flags	11

1.8	Summary table of substitution and forward estimation types	13

2	SUBSTITUTION FOR METERING INSTALLATION TYPES 1 - 4	14

2.1	Application of clause 2	14

2.2	Type 1 - 4 substitution rules	14

2.3	Type 1 - 4 substitution types	15

3	SUBSTITUTION AND forward ESTIMATION FOR METERING INSTALLATION TYPE 4A and 5	19

3.1	Application of clause 3	19

3.2	Type 4A and 5 substitution and forward estimation rules	19

3.3	Type 4A and 5 substitution and forward estimation types	20

4	SUBSTITUTION AND forward ESTIMATION FOR METERING INSTALLATION TYPE 6	23

4.1	Application of clause 4	23

4.2	Type 6 substitution and forward estimation rules	23

4.3	Type 6 substitution and forward estimation types	24

5	SUBSTITUTION AND FORWARD ESTIMATION FOR METERING INSTALLATION TYPE 7	27

5.1	Application of clause 5	27

5.2	Type 7 substitution rules	27

5.3	Type 7 substitution and forward estimation types	28

6	GENERAL DATA VALIDATION REQUIREMENTS	29

6.1	Validation requirements for metering installations types 1 - 7	29

6.2	Validation of interval metering data alarms for metering installation types 1 - 5	29

7	VALIDATION WITHIN THE METER READING PROCESS	30

7.1	Application of clause 7	30

7.2	Validations to be performed for type 4A and 5 metering data collection	30

7.3	Validations to be performed for type 6 metering data collection	30

8	VALIDATION WITHIN THE REGISTRATION PROCESS	31

8.1	General	31

8.2	Application of clause 8	31

8.3	Validation in the registration process for type 1 – 4 metering installations	31

8.4	Validation in the registration process for type 4A and 5 metering installations	32

8.5	Validation in the registration process for type 6 metering installations	32

8.6	Validation in the registration process for type 7 metering installations	33

9	VALIDATION OF METERING DATA TYPE 1 - 4	34

9.1	General	34

9.2	Application of clause 9	34

9.3	Validations to be performed for all metering installations	34

9.4	Validations to be performed for metering installations with check metering or partial check metering	35

10	VALIDATION OF METERING DATA TYPE 4A and 5	38

10.1	Application of clause 10	38

10.2	Validations to be performed for type 4A and 5 metering installations with CTs	38

10.3	Validations to be performed for whole current type 4A and 5 metering installations	39

11	VALIDATION OF METERING DATA TYPE 6	41

11.1	Application of clause 11	41

11.2	Validations to be performed for type 6 metering installations	41

12	VALIDATION OF METERING DATA TYPE 7	42

12.1	Application of clause 12	42

12.2	Validations to be performed for type 7 metering installations	42

13	LOAD PROFILING – CONVERSION OF ACCUMULATED METERING DATA	43

13.1	Application of clause 13	43

13.2	Requirement for load profiling	43

13.3	Controlled load profile	44

13.4	Net system load profile	51

13.5	Start dates and end dates	53

14	UNMETERED LOADS – DETERMINATION OF metering data	54

14.1	Application of clause 14	54

14.2	Requirement to produce calculated metering data	54

14.3	Controlled unmetered loads	56

14.4	Other unmetered loads	60

14.5	ON delay and OFF delay	62

14.6	Traffic signal dimming	66

15	Substitution READS for transfer	71

15.1	Application of clause 15	71

15.2	Type 5 metering installations	71

15.3	Type 6 metering installations	71

# General

## Introduction

### The Rules define the obligations and responsibilities of Registered Participants in the National Electricity Market relating to the provisioning and operation of a metering installation and to the processes involved in the collection of metering data.

### This document specifies the:

#### Metering Data Provider’s obligations concerning the validation, substitution and forward estimation of metering data to satisfy the Rules and metrology procedure obligations of AEMO and the metering coordinator.  These relate to all metering installations for first-tier loads and second-tier loads for which the Metering Data Provider has an obligation, as specified in AEMO’s Market Settlement and Transfer Solution (MSATS); and

#### The process of collating or determining type 6 and 7 metering installation data into trading intervals.

### These requirements must be read in conjunction with the relevant AEMO service level procedures for Metering Data Providers.

### As part of any substitution and forward estimation data quality analysis, AEMO will be undertaking periodical performance monitoring and benchmarking of all accredited Metering Data Providers.

## References

### Metrology Procedure: Part B makes reference to the documents listed in this section:

#### Metrology procedure: Part A National Electricity Market.

#### Service Level Procedure: Metering Data Provider Services Category D and C for Metering Installation Types 1, 2, 3, 4, 5, 6 and 7.

#### Meter Data File Format Specification (ME_MA001).

#### Meter Churn Data Management Procedure (ME_MA1818).

## Definitions

### A reference to a document or a provision of a document includes an amendment or supplement to, or replacement or novation of, that document or that provision of that document.

### Words in this document and in Metrology Procedure: Part A that are shown in italics have the meaning specified in clause 1.7 of Metrology Procedure: Part A or, if they are not specified in this clause, they have the meaning specified in the Rules.

## General data validation, substitution and forward estimation requirements

### The principles to be applied to metering data validation, substitution and forward estimation tasks for all areas of the National Electricity Market include the following:

#### The metering coordinator or AEMO (as appropriate) must ensure that the metering data associated with a metering installation, accurately reflects the energy flow within the respective metering installation.

#### The Metering Data Provider must authenticate all validation failures to ensure metering data accurately reflects the energy flow.

#### The Metering Data Provider must validate all substitutions and forward estimations before metering data is delivered to AEMO and Registered Participants.

#### The metering coordinator must coordinate the resolution of issues arising from the non-performance of metering systems, including any liaison with associated Retailers, Local Network Service Provider(s), Metering Provider(s) and Metering Data Provider(s).  The metering coordinator must respond promptly to requests for remedial action from the Metering Data Provider or AEMO.

#### The timeliness of a Metering Data Provider’s detection of metering data errors and the metering coordinator’s response to metering installation malfunctions may place retailers at financial risk.  The metering coordinator must, therefore, co-ordinate a response within the time frames required by the Rules and the metrology procedure and accordingly inform the affected Market Participants, so that they might implement remedial action to minimise their trading risks.

#### The Metering Data Provider must notify all affected Registered Participants of any metering data substitution or forward estimation carried out.

#### The Metering Data Provider must identify metering data errors resulting from data collection and processing operations using metering data validation processes in accordance with the metrology procedure.  The Metering Data Provider must apply metering data substitution processes in accordance with these procedures including any default substitution procedures agreed to with the metering coordinator necessary to ensure that metering data is delivered to AEMO and Registered Participants.

## Metering data substitution requirement

### The Metering Data Provider must undertake substitutions on behalf of AEMO or the metering coordinator, as appropriate, in a manner that is consistent with the metrology procedure.  Substitutions may be required in the following circumstances;

#### Where the system or equipment that supports the remote or manual collection of metering data has failed or is faulty.

#### Where the metering installation for a connection point has failed or is removed from service.

#### To enable timely provision of metering data to AEMO for settlements purposes.

#### In situations where metering data has been irretrievably lost.

#### Where the metering data obtained is found to be erroneous or incomplete.

#### Where metering data has not completed validation within the registration process with the Metering Provider as detailed in clause 8.

#### Where metering data has failed or has not completed the validation process.

#### Where metering data cannot be obtained in the performance time frames required for the data period in question:

##### Type 1, 2, 3 and 4 metering data must be substituted if metering data cannot be obtained to meet either settlement time frames or the required performance of the applicable service level procedures.

##### Type 4A, 5 and 6 metering data must be substituted if metering data cannot be obtained on or within the expected time frame of the next scheduled reading date for a connection point.  Any historical or previous forward estimated metering data must be replaced with substituted metering data.

#### When an inspection or test on the metering installation establishes that a measurement error exists due to a metering installation fault.

#### When the affected parties have all agreed and subsequently direct the Metering Data Provider that a previous substitution is in error and that a re-substitution of metering data is required.  Where the parties cannot reach agreement, the Rules’ dispute resolution procedures apply, and the existing substituted data should remain until the resolution of the dispute is achieved.

#### When an inspection or test on the respective algorithms, inventory table, load table or on/off table for type 7 metering data establishes that an error exists in the metering data calculation or when a more accurate inventory table becomes available.

#### Where the metering data calculation has failed the validation tests for a metering installation type 7.

#### In situations involving meter churn as required by the Meter Churn Data Management Procedure.

#### In response to customer transfers authorised by a participating jurisdiction or retailer of last resort events, as outlined in clause 15.

## Metering data forward estimation requirement

### The Metering Data Provider must undertake forward estimations on behalf of the metering coordinator in a manner that is consistent with the metrology procedure.  Forward estimations may be required in the following circumstances;

#### Routinely for a period that is equal to or just greater than the period to the next scheduled reading date or another forward period.

#### In response to customer transfers authorised by a participating jurisdiction or retailer of last resort events, as outlined in clause 15.

### Where the current published scheduled reading date has changed due to a revised scheduled reading route, the Metering Data Provider must replace any existing forward estimated metering data with new forward estimated metering data related to the revised next scheduled reading date.

### This clause only applies to a jurisdiction as specified in the following table:

| Jurisdiction | Variation in accordance with jurisdictional policy |
| --- | --- |
| Victoria | Despite Clause 1.6.1(a) and 1.6.2, where metering data for a type 5 metering installation is collected more frequently than required by Metrology Procedure Part A Clause 3.3.9 (as allowed by Metrology Procedure Part A Clause 3.8.4A [Vic]), estimations need not be provided routinely or as a result of a change to the current published scheduled reading date. Estimations must, however, be provided where necessary to meet the data requirements of Schedule 8 of the Service Level Requirements for Metering Data Collection, Processing and Delivery Services for Metering Provider category 5D, 6D and 7D, but are not required to be for a period to the next scheduled reading date |

#### Note:  The effective date of this Jurisdictional provision is 1 July 2009. The review date of this Jurisdictional provision is 31 December 2017.

## Metering data quality flags

### Metering Data Providers must assign the relevant metering data quality flags to metering data as follows:

#### A - For validated and accepted actual metering data recovered from the metering installation.

#### S – For any substituted metering data that is considered temporary and may be replaced by actual metering data at some time.  Metering data substitutions apply to historical date/time periods at the time of substitution.

#### E – For any forward estimated metering data that is considered temporary and may be replaced by actual metering data or substituted metering data at some time.  Metering data forward estimations apply to a period that has an end date/time in the future.

#### F – For substitutions that are of a permanent or final nature and subject to clause 1.7.3, the metering data would not be replaced by actual metering data at any time.

#### N – This quality flag is only utilised within the participant interval metering data file for instances where no metering data exists within the metering data service database for the periods concerned.

### Unless specified otherwise within the metrology procedure, Metering Data Providers must apply the following quality flag rules in the management of metering data:

#### A metering data can only be replaced with A, S or F metering data.

#### S metering data can only be replaced with A, S or F metering data.

#### E metering data can only be replaced with A, E, S or F metering data.

#### F metering data can only be replaced with F metering data as per clause 1.7.3 (f) or A metering data as per clause 1.7.3(b) or 1.7.3(h).

### The Metering Data Provider must manage final substitutions as follows:

#### The Metering Data Provider must undertake final metering data substitutions in the instance where a notice has been received from either the metering coordinator or the Metering Provider detailing a failure of the metering installation and that actual metering data cannot be obtained.

#### In the rare instance where validated actual metering data is unexpectedly recovered from the metering installation and a final substitution has been undertaken in accordance with clause 1.7.3(a), the Metering Data Provider must replace the final substituted metering data with the actual metering data and maintain a record of the reason and instance.

#### In the instance where the Metering Data Provider must undertake final metering data substitutions as identified within the Meter Churn Data Management Procedure.

#### In the instance where the Metering Data Provider has received a notice that the financially responsible Market Participant, the Local Retailer and the Local Network Service Provider have all agreed that the metering data is erroneous and that a final metering data substitution is required.

#### In the instance of a retailer of last resort event or where a Jurisdiction has requested AEMO to undertake authorised customer transfers requiring the provision of metering data substitutions and final readings, in accordance with clause 15.

#### The Metering Data Provider may undertake to replace an existing final metering data substitution with a new final metering data substitution in accordance with the metrology procedure.

#### In the instance where the Metering Data Provider has found previous validated actual metering data to be erroneous, then the actual metering data must be replaced with a final metering data substitution.

#### The Metering Data Provider may replace type 6 final metering data substitutions with actual metering data that spans consecutive metering readings on agreement with the financially responsible Market Participant, the Local Retailer and the Local Network Service Provider.

## Summary table of substitution and forward estimation types

| Type | EST or SUB | Installation Type | Short Descriptor | Page |
| --- | --- | --- | --- | --- |
| Type 11 | SUB | 1 - 4 | Check | 14 |
| Type 12 | SUB | 1 - 4 | Calculated | 15 |
| Type 13 | SUB | 1 - 4 | SCADA | 15 |
| Type 14 | SUB | 1 - 4 | Like Day | 15 |
| Type 15 | SUB | 1 - 4 | Ave Like Day | 16 |
| Type 16 | SUB | 1 - 4 | Agreed | 17 |
| Type 17 | SUB | 1 - 4 | Linear | 17 |
| Type 18 | SUB | 1 - 4 | Alternate | 17 |
| Type 19 | SUB | 1 - 4 | Zero | 17 |
| Type 51 | SUB or EST | 4A, 5 | Previous Year | 19 |
| Type 52 | SUB or EST | 4A, 5 | Previous Read | 19 |
| Type 53 | SUB | 4A, 5 | Revision | 20 |
| Type 54 | SUB | 4A, 5 | Linear | 20 |
| Type 55 | SUB | 4A, 5 | Agreed | 20 |
| Type 56 | SUB or EST | 4A, 5 | Prior to First Read - Agreed | 20 |
| Type 57 | SUB or EST | 4A, 5 | Customer Class | 21 |
| Type 58 | SUB or EST | 4A, 5 | Zero | 21 |
| Type 61 | SUB or EST | 6 | Previous Year | 23 |
| Type 62 | SUB or EST | 6 | Previous Read | 23 |
| Type 63 | SUB or EST | 6 | Customer Class | 24 |
| Type 64 | SUB | 6 | Agreed | 24 |
| Type 65 | SUB or EST | 6 | ADL | 24 |
| Type 66 | SUB | 6 | Revision | 24 |
| Type 67 | SUB | 6 | Customer Read | 24 |
| Type 68 | SUB or EST | 6 | Zero | 25 |
| Type 71 | SUB | 7 | Recalculation | 27 |
| Type 72 | SUB | 7 | Revised Table | 27 |
| Type 73 | SUB | 7 | Revised Algorithm | 27 |
| Type 74 | SUB | 7 | Agreed | 27 |
| Type 75 | EST | 7 | Existing Table | 27 |

# SUBSTITUTION FOR METERING INSTALLATION TYPES 1 - 4

## Application of clause 2

### The requirements of clause 2.2 and clause 2.3 are applicable to all jurisdictions within the National Electricity Market and apply to metering installation types 1, 2, 3 and 4.

### The requirements of clause 2.2 and clause 2.3 are applicable to all metering data substitution types to be undertaken by Metering Data Providers accredited for the collection, processing and delivery of metering data from metering installation types 1, 2, 3 and 4.

### For all metering data substitutions undertaken under clause 2.2 and clause 2.3, for metering installations types 1, 2, 3 and 4, the Metering Data Provider must ensure the selected metering data substitution values correctly align with the adjoining intervals of metering data and that any intervals of metering data adjacent to the substituted period are valid.

## Type 1 - 4 substitution rules

### The Metering Data Provider must carry out all metering data substitutions in accordance with the metrology procedure.

### The Metering Data Provider must obtain clear and concise identification as to the cause of any missing or erroneous metering data for which substitutions are required.

### The Metering Data Provider must undertake to do a type 11 substitution and use metering data obtained from any check metering installation associated with the connection point as the first choice considered for the source of metering data for any substitutions undertaken.

### Subject to clause 2.2.5, SCADA metering data, where available, may be used by the Metering Data Provider as check metering data for substitutions.

### Metering Data Providers may only undertake substitution type 13 where substitution type 11 and type 12 are not applicable or cannot be carried out.

### For connection points classified as Generators or Market Small Generation Aggregators:

#### Metering Data Providers may directly undertake type 11, type 12 or type 13 substitutions as a consequence of missing or erroneous metering data that has failed validation.

#### Metering Data Providers may undertake type 16 (Agreed Method) or Type 18 (Alternate) substitutions following consultation and agreement with the Generator or Market Small Generation Aggregator that the substituted metering data is an accurate reflection of the interval metering data concerned.

#### In the situation where metering data cannot be recovered from the metering installation or substituted in accordance with this clause 2.2.6 within the required metering data delivery performance time frames, the Metering Data Provider must undertake type 19 substitutions as an interim until metering data can be recovered from the metering installation or substituted.

### Metering Data Providers may only undertake substitution types 14, 15, 16, 17, 18, or 19 where substitution types 11, 12 and 13 are not applicable or cannot be carried out.

### Metering Data Providers may perform all metering data substitution types except type 16 or type 18 without prior agreement from the affected parties.  Metering Data Providers may however undertake to change the quality flag to an existing type 16 or type 18 substitution without seeking further agreement from the affected parties.

### The Metering Data Provider must notify the Local Network Service Provider, Local Retailer and the financially responsible Market Participant for the connection point of any metering data substitution within two business days of the metering data substitution being carried out by the Metering Data Provider.  Notification is to be achieved via the participant metering data file as detailed within the service level procedures.

### Where a metering installation malfunction is a failure of the remote acquisition system, and the metering coordinator and Metering Provider cannot repair the data communications within the periods specified in the Rules, (two business days for type 1, 2 and 3 metering installations and 10 business days for type 4 metering installations) then the Metering Data Provider must:

#### Request from the metering coordinator, the provision of a manual download of metering data from the metering installation in the time frames to meet metering data delivery requirements; and

#### Where the malfunction includes a failure of the meter to correctly record interval energy data and the metering coordinator has acquired an exemption to repair the metering installation, then the Metering Data Provider must substitute for the missing metering data in accordance with the metrology procedure.

### The Metering Data Provider must ensure that all metering data substitutions are replaced with actual metering data when that metering data becomes available.

## Type 1 - 4 substitution types

### Type 11 - Check Data

#### The Metering Data Provider must use interval metering data obtained from another metering installation that has been identified by the Metering Provider as being the check metering installation for that metering point.  The metering data used must be for the same trading intervals as the trading intervals that are being substituted.  Metering installations of this type include but are not limited to:

#### The metering installation and check metering installation are installed at the same connection point.

#### The metering installation and check metering installation are installed on different ends of a transmission line where the difference due to line losses can be accurately determined.

#### Metering installations across a parallel set of feeders having similar line impedances between a common set of busbars.

### Type 12 - Calculated

#### The Metering Data Provider must calculate the interval metering data values to be substituted where they relate to a single unknown feed to a node, based on the other known energy flows to or from that node.

### Type 13 - SCADA

#### The Metering Data Provider must use EMS or SCADA data, as provided by AEMO in the agreed format, for substitution purposes, which originates from a similar measurement point as the meter.

#### EMS or SCADA data may be data which is inferior in accuracy or resolution and which is in a dissimilar format to the metering data, (e.g. 30 Min. Demand values).  The Metering Data Provider may have to adjust the data in both magnitude and form in order that the substitution is acceptable.

#### In any instance where SCADA data is to be used for substitution, both the provided E channel and B channel SCADA data streams must be used.

### Type 14 - Like Day

#### The Metering Data Provider must substitute for the missing or erroneous metering data using the nearest equivalent day or like day method, as detailed in Table 1 below.

| TYPE 14 | Table 1 |
| --- | --- |
| Substitution Day | Nearest Equivalent Day or Like Day (in order of availability) |
| Monday | Monday ## |
| Tuesday | Tuesday## Wednesday## Thursday## Wednesday# Thursday# |
| Wednesday | Wednesday## Tuesday# Thursday## Thursday# Tuesday## |
| Thursday | Thursday## Wednesday# Tuesday# Wednesday## Tuesday## |
| Friday | Friday## |
| Saturday | Saturday## |
| Sunday | Sunday## |
| Metering data substitutions for like day to be as detailed above, unless: - (a) No metering data is available on the first listed day, then the next listed preferred day is to be used. If there is no other suitable listed day, or no metering data is available on any of the listed days then type 15 substitution must be used. - (b) The substitution day was a public holiday, in which case the most recent Sunday is to be used. - (c) The substitution day was not a public holiday and the listed day is a public holiday, then the next listed preferred day that is not a public holiday is to be used. # Occurring in the same week as the substitution day. ## Occurring in the week preceding that in which the substitution day occurs. |  |

### Type 15 - Average Like Day

#### The Metering Data Provider may substitute for the missing or erroneous metering data using the average like day method, as detailed in Table 2 below.

TYPE 15 Table 2

The interval metering data to be substituted will be calculated using an average of the metering data from each corresponding interval from the preceding four weeks, or part thereof. This averaging technique may be applied in either of the following ways: - Where the averaged interval metering data is used to provide the value for the interval(s) requiring substitution. - Where the averaged interval metering data is used to provide the profile and is scaled to a pre-determined consumption value for the interval(s) to be substituted. Type 15 substitutions must not be used for public holidays.

### Type 16 - Agreed Method

#### Where the Metering Data Provider is required to undertake a metering data substitution for any period greater than seven days, consultation and agreement must be obtained from the financially responsible Market Participant, the Local Retailer and the Local Network Service Provider for the connection point as to the metering data substitution to be performed.  This may include changes to existing metering data substitutions for any period which were carried out where the affected parties have directed that as a result of site or customer specific information, the original metering data substitutions are in error and a correction is required.

### Type 17 - Linear Interpolation

#### The Metering Data Provider may substitute metering data for consecutive interval periods up to, but not exceeding two hours, by using simple linear interpolation.

### Type 18 - Alternate

#### The Metering Data Provider may use an alternate method of metering data substitution subject to an agreement between the financially responsible Market Participant, the Local Retailer and the Local Network Service Provider for the connection point.  The specifics of this substitution type may involve a globally applied method or a method where an adjusted profile is used to take into account local conditions which affect consumption (e.g. local holiday or customer shutdown), or where alternate metering data may be able to be used for quality checks and minor adjustments of an estimated profile, such as using meter register data.

### Type 19 - Zero

#### The Metering Data Provider must undertake metering data substitutions of ‘zero’ where:

#### Either the Local Network Service Provider or the Metering Provider has informed the Metering Data Provider of a de-energised connection point or an inactive meter and where the consumption is known to be zero; or

#### As specified within the Meter Churn Data Management Procedure; or

#### Metering data substitutions are applicable for connection points classified as Generators or Market Small Generation Aggregators in accordance with clause 2.2.6(c).

# SUBSTITUTION AND forward ESTIMATION FOR METERING INSTALLATION TYPE 4A and 5

## Application of clause 3

### The requirements of clause 3.2 and clause 3.3 apply to the jurisdictions of Victoria, New South Wales, South Australia, Australian Capital Territory, Queensland, and Tasmania.

### The substitution and forward estimation types, as detailed within clause 3.2 and clause 3.3, are to be undertaken by Metering Data Providers accredited for the collection, processing and delivery of metering data from a metering installation type 5.

### For all metering data substitutions and forward estimations undertaken under clause 3.2 and clause 3.3, for metering installations type 5, the Metering Data Provider must ensure the selected metering data substitution values correctly align with the adjoining interval metering data and that any interval metering data adjacent to the substituted period is valid.

## Type 4A and 5 substitution and forward estimation rules

### The Metering Data Provider must carry out all metering data substitutions and forward estimations in accordance with the metrology procedure.

### The Metering Data Provider must ensure that all metering data substitutions and forward estimations are replaced with actual metering data if and when that metering data becomes available.

### The Metering Data Provider must obtain clear and concise identification as to the cause of any missing or erroneous metering data for which metering data substitutions are required.

### The Metering Data Provider must only use type 56 or type 57 substitutions or forward estimations where the historical metering data does not support the application of a type 51 or type 52 substitution or forward estimation.

### Subject to clause 3.2.4 the Metering Data Provider must only apply the following substitution and forward estimation types:

#### Substitutions may be type 51, 52, 53, 54, 55, 56, 57 or 58.

#### Forward estimations may be type 51, 52, 56, 57 or 58.

### The Metering Data Provider must notify the Local Network Service Provider, Local Retailer and the financially responsible Market Participant for the connection point of any metering data substitution or forward estimation within two business days of the metering data substitution or forward estimation being carried out by the Metering Data Provider.  Notification is achieved via the participant metering data file as detailed within the service level procedures.

### Metering Data Providers may not perform type 53 or type 55 substitutions or type 56 substitutions or forward estimations without prior agreement with the affected parties.  Metering Data Providers may however undertake to change the quality flag to an existing type 53 or type 55 substitution or type 56 substitution or forward estimation without seeking further agreement from the affected parties.

## Type 4A and 5 substitution and forward estimation types

### Type 51 - Previous Years Method (Nearest Equivalent Day or Like Day)

#### The Metering Data Provider must provide a substitution or forward estimation for the metering data using the metering data from the nearest equivalent day or like day from the same, or similar, meter reading period in the previous year.  The nearest equivalent day or like day is to be determined from Table 3 below.

### Type 52 - Previous Meter Reading Method (with the Nearest Equivalent Day or Like Day method)

#### The Metering Data Provider must provide a substitution or forward estimation for the metering data using the metering data from the nearest equivalent day or like day from the previous meter reading period.  The nearest equivalent day or like day is to be determined from Table 3 below.

| Type 51 or 52 | Table 3 |
| --- | --- |
| Substitution or Forward Estimation Day | Nearest Equivalent Day or Like Day (in order of availability) |
| Monday | Monday## Monday # |
| Tuesday | Tuesday## Wednesday## Tuesday# Wednesday# |
| Wednesday | Wednesday## Tuesday## Thursday## Wednesday# Thursday# Tuesday# |
| Thursday | Thursday## Wednesday## Tuesday## Thursday# Wednesday# Tuesday# |
| Friday | Friday## Friday# |
| Saturday | Saturday## Saturday# |
| Sunday | Sunday## Sunday# |
| Metering data substitutions or forward estimations for like day to be as detailed above, unless: - No metering data is available on the first listed day, then the next listed preferred day is to be used. If there is no other suitable day, or no metering data is available on any of the listed days then Type 52 must be used. - The substitution or forward estimation day was a public holiday, in which case the most recent Sunday is to be used. - The substitution or forward estimation day was not a public holiday and the listed day is a public holiday, then the next listed preferred day that is not a public holiday, Saturday or Sunday is to be used. |  |
| ## For type 51 utilise metering data from the corresponding week in the previous year. ## For type 52 utilise metering data from the corresponding week of the previous meter reading period. # For type 51 utilise metering data from the week preceding the corresponding week in the previous year. # For type 52 utilise metering data occurring in the week preceding the corresponding week of the previous meter reading period. |  |

#### Alternatively, the Metering Data Provider must provide substitution or forward estimation metering data using the average like day method, as detailed in Table 4.

TYPE 52 (Alternate) Table 4

The interval metering data, for which a substitution or forward estimation is to be provided, will be calculated using an average of the metering data from each corresponding interval from the preceding four weeks, or part thereof. This averaging technique may be applied in either of the following ways: - Where the averaged interval metering data is used to provide the value for the interval requiring substitution or forward estimation. - Where the averaged interval metering data is used to provide the profile and are scaled to a pre-determined consumption value for the interval(s) that are the subject of substitution or forward estimation. Type 52 substitutions or forward estimations must not be used for public holidays

### Type 53 - Revision of Substituted Metering Data

#### The Metering Data Provider must re-substitute or change previously substituted metering data prior to the actual meter reading or prior to the second revision in the AEMO settlements timetable (whichever occurs first), where the financially responsible Market Participant, the Local Retailer and the Local Network Service Provider have agreed, on the basis of site or customer specific information, that the original substituted metering data is in error and a correction is required.

### Type 54 - Linear Interpolation

#### The Metering Data Provider may substitute metering data for consecutive interval periods up to, but not exceeding two hours, by using simple linear interpolation.

### Type 55 - Agreed Substitution Method

#### The Metering Data Provider may undertake to use another method of metering data substitution (which may be a modification of an existing substitution type), where none of the existing substitution types is applicable, subject to an agreement between the financially responsible Market Participant, the Local Retailer and the Local Network Service Provider for the connection point.  The specifics of this substitution type may involve a globally applied method.

### Type 56 - Prior To First Reading - Agreed Method

#### Prior to the first actual meter reading and where no previous metering data history exists for the connection point, the Metering Data Provider may provide a substitution or forward estimation for the interval metering data using a method agreed between the financially responsible Market Participant, the Local Retailer and the Local Network Service Provider.

### Type 57 - Prior to First Reading - Customer Class Method

#### Prior to the first actual meter reading and where no previous metering data history exists for the connection point, the Metering Data Provider may provide a substitution or forward estimation for the metering data based on the given average daily load.  The interval metering data must be profiled to suit the relevant customer class.  Metering Data Providers electing to undertake this type of substitution or forward estimation must develop a suite of profiles acceptable to the metering coordinator for use and application.

### Type 58 - Zero

#### The Metering Data Provider must undertake metering data substitutions or forward estimations of ‘zero’ where:

#### Either the Local Network Service Provider or the Metering Provider has informed the Metering Data Provider of a de-energised connection point or an inactive meter and where the consumption is known to be zero; or

#### As specified within the Meter Churn Data Management Procedure.

# SUBSTITUTION AND forward ESTIMATION FOR METERING INSTALLATION TYPE 6

## Application of clause 4

### The requirements of clause 4.2 and clause 4.3 apply to the jurisdictions of Victoria, New South Wales, South Australia, Australian Capital Territory, Queensland, and Tasmania.

### The metering data substitution and forward estimation types as detailed within clause 4.2 and clause 4.3 are to be undertaken by Metering Data Providers accredited for the collection, processing and delivery of metering data from a metering installation type 6.

## Type 6 substitution and forward estimation rules

### The Metering Data Provider must carry out all metering data substitutions and forward estimations in accordance with the metrology procedure.

### The Metering Data Provider must replace all metering data forward estimations with either actual or substituted metering data:

#### When actual metering data covering all or part of the forward estimation period is obtained; or

#### When the next scheduled meter reading was unable to be undertaken, the Metering Data Provider must replace the forward estimated metering data with substituted metering data with a quality flag of F (final substitution).

### Any final substituted metering data provided by the Metering Data Provider must be re-validated, updated or recalculated by the Metering Data Provider when:

#### The value of the metering data obtained at the next actual meter reading is found to be less than the previous final substitution; or

#### The final substituted value is disputed and following consultation and agreement from the financially responsible Market Participant, the Local Retailer and the Local Network Service Provider for the connection point a new agreed value as per clause 4.3.4 (type 64) must be provided.

### The Metering Data Provider must obtain clear and concise identification as to the cause of any missing or erroneous metering data for which metering data substitutions are required.

### Where the scheduled meter reading frequency is less frequent than monthly, the Metering Data Provider may only use a type 62 substitution or forward estimation method when metering data from the same, or similar, meter reading period last year (i.e. type 61) is not available.

### The Metering Data Provider may use type 63 substitutions or forward estimations only when the metering data from the same, or similar, meter reading period last year and metering data from the previous meter reading period is not available (i.e. when type 61 and type 62 substitution or forward estimation methods cannot be used).

### The Metering Data Provider may use type 65 substitutions or forward estimations only when the metering data from the same, or similar, meter reading period last year or the metering data from the previous meter reading period is not available (i.e. when type 61 and type 62 substitution or forward estimation methods cannot be used).

### The Metering Data Provider must only use a type 67 substitution when:

#### Directed by the metering coordinator; and

#### Not expressly disallowed in the jurisdiction concerned; and

#### The end-use customer provided meter reading meets the validation rules for that data stream; and

#### The Metering Data Provider has no actual metering data for the scheduled reading date for this connection point.

### Subject to clause 4.2.5, clause 4.2.6, clause 4.2.7 and clause 4.2.8 the Metering Data Provider may apply the following substitution and forward estimation types:

#### Substitutions may be type 61, 62, 63, 64, 65, 66, 67 or 68.

#### Forward estimations may be type 61, 62, 63, 65 or 68.

### Metering Data Providers may not perform type 64 or type 66 substitutions without prior agreement with the financially responsible Market Participant, the Local Retailer and the Local Network Service Provider for the connection point.  Metering Data Providers may however undertake to change the quality flag to an existing type 64 or type 66 substitution without seeking further agreement from the financially responsible Market Participant, the Local Retailer and the Local Network Service Provider for the connection point.

### The Metering Data Provider must notify the Local Network Service Provider, Local Retailer and the financially responsible Market Participant for the connection point of any metering data substitution or forward estimation within two business days of the metering data substitution or forward estimation being carried out by the Metering Data Provider.  Notification is achieved via the participant metering data file as detailed within the service level procedures.

## Type 6 substitution and forward estimation types

### Type 61 - Previous Year Method (Average Daily Consumption method)

#### The Metering Data Provider must provide a substitution or forward estimation of the meter reading by calculating the energy consumption as per the following formula:

#### Energy Consumption = ADCLY  number of days required

#### where

#### ADCLY = average daily consumption from the same or similar meter reading period last year.

### Type 62 - Previous Meter Reading Method (Average Daily Consumption Method)

#### The Metering Data Provider must provide a substitution or forward estimation of the meter reading by calculating the energy consumption as per the following formula:

#### Energy Consumption = ADCPP  number of days required.

#### where

#### ADCPP = average daily consumption from the previous meter reading period.

### Type 63 Customer Class Method

#### The Metering Data Provider must provide a substitution or forward estimation of the meter reading by calculating the energy consumption as per the following formula:

#### Energy Consumption = ADCCC  number of days required

#### where

#### ADCCC = average daily consumption for this customer class with the same type of usage.

### Type 64 - Agreed Method

#### The Metering Data Provider may undertake to use another method of metering data substitution (which may be a modification of an existing substitution type), where none of the existing substitution types is applicable, subject to an agreement between the financially responsible Market Participant, the Local Retailer and the Local Network Service Provider for the connection point.  The specifics of this substitution type may involve a globally applied method.

### Type 65 – ADL Method

#### The Metering Data Provider must provide a substitution or forward estimation of the meter reading by calculating the energy consumption in accordance with the following formula:

#### Energy Consumption = Average Daily Load  number of days required.

### Type 66 - Revision of Substituted Metering Data

#### The Metering Data Provider must re-substitute or change previously substituted metering data, prior to an actual meter reading or prior to the second revision in the AEMO settlements timetable (whichever occurs first), where the financially responsible Market Participant, the Local Retailer and the Local Network Service Provider have agreed to revise the original substituted metering data, on the basis of site or customer specific information.

### Type 67 – End-use customer Reading

#### Subject to clause 4.2.8, the Metering Data Provider must substitute any previously substituted or forward estimated metering data based directly on an end-use customer provided meter reading.

### Type 68 - Zero

#### The Metering Data Provider must undertake data substitutions or forward estimations of ‘zero’ where:

#### Either the Local Network Service Provider or the Metering Provider has informed the Metering Data Provider of a de-energised connection point or an inactive meter and where the consumption is known to be zero; or

#### As specified within the Meter Churn Data Management Procedure.

# SUBSTITUTION AND FORWARD ESTIMATION FOR METERING INSTALLATION TYPE 7

## Application of clause 5

### The requirements of clause 5.2 and clause 5.3 apply to the jurisdictions of Victoria, New South Wales, South Australia, Australian Capital Territory, Tasmania and Queensland.

### The substitution and forward estimation types detailed in clause 5.2 and clause 5.3 are to be undertaken by Metering Data Providers accredited for the calculation and delivery of metering data from a metering installation type 7.

## Type 7 substitution rules

### The Metering Data Provider must carry out all metering data substitutions and forward estimations in accordance with the metrology procedure.

### The Metering Data Provider must obtain clear and concise identification as to the cause of any missing or erroneous calculated metering data for which metering data substitutions are required.

### The Metering Data Provider must ensure that all metering data substitutions and forward estimations are based on calculated metering data and not on any previous substitutions.

### The Metering Data Provider must base calculated metering data for type 7 metering installations on inventory table data as follows:

#### Where the inventory table has not been updated for the period concerned, calculated metering data must be based on the most recent available information and provided as a forward estimated value; and

#### Where the inventory table is correct for the period concerned, the calculated metering data must be provided as an actual value; and

#### Where the inventory table in (b) above has a subsequent update for the period concerned, the calculated metering data must be provided as a substituted value.

### Subject to clause 5.2.4, the Metering Data Provider may apply the following substitution and forward estimations types:

#### Substitutions may be type 71, 72, 73, or 74.

#### Forward estimations must be type 75.

### The Metering Data Provider must notify the Local Network Service Provider, Local Retailer and the financially responsible Market Participant for the connection point of any calculated metering data substitution by the Metering Data Provider within two business days of the calculated metering data substitution being carried out by the Metering Data Provider.  Notification is achieved via the participant metering data file as detailed within the sevice level procedures.

### The Metering Data Provider must flag all calculated metering data substitutions as final (F).

### Metering Data Providers may not perform a type 74 substitution without prior agreement with the affected parties.

## Type 7 substitution and forward estimation types

### Type 71 - Recalculation

#### The Metering Data Provider must substitute calculated metering data with the calculated metering data obtained by a recalculation based on the current inventory tables, load tables and on/off tables.

### Type 72 - Revised Tables

#### Where the error in the calculated metering data is due to errors in the inventory table, load table or on/off table, the Metering Data Provider must substitute calculated metering data obtained by a recalculation based on the most recent inventory tables, load tables and on/off tables in which there were no errors.

### Type 73 - Revised Algorithm

#### Where the error in the calculated metering data is due to an error in the algorithm, the Metering Data Provider must substitute the most recent calculated metering data for which there was no error.

### Type 74 - Agreed Method

#### The Metering Data Provider may use another method of calculate metering data substitution (which may be a modification of an existing substitution type), where none of the existing substitution types is applicable, subject to an agreement between the financially responsible Market Participant, the Local Retailer and the Local Network Service Provider for the connection point.  The specifics of this substitution type may involve a globally applied method.

### Type 75 – Existing Table

#### The Metering Data Provider must provide a forward estimate for the calculated metering data based on the most recent inventory table information until such time as an updated inventory table is received for the period concerned.

# GENERAL DATA VALIDATION REQUIREMENTS

## Validation requirements for metering installations types 1 - 7

### Metering Data Providers are requested to manage systems and processes on the basis that:

#### Registration information given may not be inherently correct;

#### Stored metering data held in the meter buffer may be subject to installation measurement error;

#### Data delivered by reading systems, (e.g. Remote reading systems, hand held readers and conversion software) may not be recovered from the field meters without corruption.

### The market use of auditable validation procedures is of critical importance and may have a direct impact on disputes.  It is essential that Metering Data Providers comply with these validation procedures and that all metering data is subjected to validation prior to delivery to AEMO and Registered Participants.

## Validation of interval metering data alarms for metering installation types 1 - 5

### The Metering Data Provider must validate interval metering data from metering installation types 1, 2, 3, 4 and 5 against the following significant metering data alarms when these are provided in the meter:

•	power failure / meter loss of supply,

•	VT or phase failure,

•	pulse overflow,

•	CRC error, and

•	time tolerance.

#### Where interval metering installations assign alarms to the data channel and or the interval metering data concerned, the Metering Data Provider must process the alarm along with the metering data as part of the required metering data validation process.

#### As a minimum requirement, the Metering Data Provider must have systems and processes in place that capture metering data alarms and process them by exception reporting.

#### The Metering Data Provider must retain all metering data alarm exception reports for audit purposes.

#### The Metering Data Provider must ensure that all metering data alarm reports are signed off and dated by the person actioning the data exception report review as part of the validation process.

#### The Metering Data Provider must validate all interval metering data with all metering data alarms prior to despatch to AEMO or Registered Participants.

#### All Metering Data Provider exception reports must provide, for all instances where the interval metering data was found to be corrupted e.g. intervals substituted, an indication of the subsequent actions undertaken by the Metering Data Provider.

# VALIDATION WITHIN THE METER READING PROCESS

## Application of clause 7

### The requirements of clause 7.2 are applicable to Metering Data Providers accredited for the provision of metering data services for type 4A and  5 metering installations.

### The requirements of clause 7.3 are applicable to Metering Data Providers accredited for the provision of metering data services for type 6 metering installations.

## Validations to be performed for type 4A and 5 metering data collection

### The Metering Data Provider responsible for the collection of metering data from type 4A and 5 metering installations must undertake the following validations within the meter reading process:

#### The meter serial number is correct against the recorded meter serial number.

#### The security of the metering installation is intact, e.g. meter seals in place and in good order.

#### The time synchronisation of the metering installation is correct to EST inclusive of any load control devices.

## Validations to be performed for type 6 metering data collection

### The Metering Data Provider responsible for the collection of metering data from type 6 metering installations must undertake the following validations within the meter reading process:

#### The value of metering data from the current meter reading ≥ the value of metering data from the previous meter reading.

#### The value of metering data from the current meter reading is valid against an expected minimum value.

#### The value of metering data from the current meter reading is valid against an expected maximum value.

#### The meter serial number is correct against the recorded meter serial number.

#### The security of the metering installation is intact, e.g. meter seals in place and in good order.

#### The time synchronisation of the metering installation is correct to EST inclusive of any load control devices.

#### The dial capacity is checked against the recorded dial capacity.

# VALIDATION WITHIN THE REGISTRATION PROCESS

## General

### This clause 8 relates to the validation of the NMI and respective metering data within the registration and connection point transfer process.  This clause 8 is also applicable to current registered connection points which have a change of metrology and/or Metering Data Provider.

## Application of clause 8

### The requirements of clause 8.3 are applicable to Metering Data Providers accredited for the provision of metering data services for type 1, 2, 3 and 4 metering installations.

### The requirements of clause 8.4 are applicable to Metering Data Providers accredited for the provision of metering data services for type 4A and 5 metering installations.

### The requirements of clause 8.5 are applicable to Metering Data Providers accredited for the provision of metering data services for type 6 metering installations.

### The requirements of clause 8.6 are applicable to Metering Data Providers accredited for the provision of metering data services for type 7 metering installations.

## Validation in the registration process for type 1 – 4 metering installations

### The Metering Data Provider must validate the metering data on registration of all type 1, 2, 3 and 4 metering installations to verify:

#### That the metering data correctly pertains to the registered metering installation, in conjunction with the Metering Provider.

#### That the magnitude and profile of the metering data is correct for the primary energy, in conjunction with the Metering Provider.

#### That all data streams are captured, in conjunction with the Metering Provider.

#### That the NMI is registered in MSATS.

#### That the NMI is correct and is within the range allocated for that Network Service Provider.

### The Metering Data Provider must undertake all validations as detailed in clause 8.3.1 following any changes to the metering installation (e.g. a meter change or current transformer (CT) ratio change).

### The validations performed by the Metering Data Provider as detailed in clause 8.3.1, must be carried out prior to the distribution of any actual interval metering data to AEMO or Registered Participants for the purposes of market settlements or billing.

## Validation in the registration process for type 4A and 5 metering installations

### The Metering Data Provider must validate the metering data on registration of all current transformer type 4A and 5 metering installations to verify:

#### That the metering data correctly pertains to the registered metering installation, in conjunction with the Metering Provider.

#### That the magnitude and profile of the metering data is correct for the primary energy, in conjunction with the Metering Provider.

#### That all data streams are captured, in conjunction with the Metering Provider.

#### That the NMI is registered in MSATS.

#### That the NMI is within the range allocated for that Local Network Service Provider.

### The Metering Data Provider must undertake all validations as detailed in clause 8.4.1 following any changes to the metering installation (such as a meter change or current transformer (CT) ratio change).

### The validations performed by the Metering Data Provider, as detailed in clause 8.4.1, must be carried out prior to the distribution of any actual interval metering data to AEMO or Registered Participants for the purposes of market settlements or billing.

### The Metering Data Provider must validate the metering data on registration of all whole current type 4A and 5 metering installations to verify:

#### That the metering data correctly pertains to the registered metering installation.

#### That the metering data acquired is of an expected magnitude and profile for the customer type.

#### That all data streams are captured.

#### That the NMI is registered in MSATS.

#### That the NMI is within the range allocated for that Local Network Service Provider.

### The validation performed by the Metering Data Provider, as detailed in clause 8.4.4, must be carried out prior to the distribution of any actual interval metering data to AEMO or Registered Participants for the purposes of market settlements or billing.

## Validation in the registration process for type 6 metering installations

### The Metering Data Provider must validate the metering data on registration of all type 6 metering installations to verify:

#### That the metering data correctly pertains to the registered metering installation.

#### That all data streams are captured.

#### That the NMI is registered in MSATS.

#### That the NMI is within the range allocated for that Local Network Service Provider.

### The Metering Data Provider must undertake all validations, as detailed in clause 8.5.1, following any changes to the metering installation (such as a meter change or current transformer (CT) ratio change).

### The validations performed by the Metering Data Provider, as detailed in clause 8.5.1, must be carried out prior to the distribution of any actual metering data to AEMO or Registered Participants for the purposes of market settlements or billing.

## Validation in the registration process for type 7 metering installations

### The Metering Data Provider must validate the calculated metering data on registration of all type 7 metered sites to verify:

#### That the inventory tables, load tables and on/off tables are complete and correct for the specifics of the type 7 metering installation.

# VALIDATION OF METERING DATA TYPE 1 - 4

## General

### For connection points where AEMO is required to appoint a Metering Data Provider to provide metering data services, AEMO may also request the respective Metering Data Provider to perform additional data validations (e.g. busbar or nodal validation) to further ensure the quality and completeness of the metering data.

### Type 1 and 2 metering installations must have a check metering installation in accordance with the Rules.

### Type 3, 4, 5 and 6 metering installations are not required to have (but may have) a check metering installation in accordance with the Rules.

## Application of clause 9

### The requirements of clause 9.3 are applicable to all Metering Data Providers accredited for the provision of metering data services for metering installation types 1, 2, 3 and 4.

### The requirements of clause 9.4 are applicable to Metering Data Providers accredited for the provision of metering data services for metering installation types 1, 2, 3 and 4 that have associated check metering installations or partial check metering installations.

## Validations to be performed for all metering installations

### The Metering Data Provider must, as a minimum, undertake the following validations on metering data within the metering data services database:

#### A check of all interval metering data against a nominated maximum value.

##### This validation is to ensure that erroneous interval metering data spikes are trapped and substituted.

##### This check may additionally be performed in the polling software.

#### A check of the maximum value of active energy and reactive energy.

##### For current transformer (CT) metering installations the maximum value is to be initially defined by the applied current transformer (CT) ratio of the metering installation.

##### For whole current metering installations the maximum rating of the meter is to be used.

#### Check against a nominated minimum value or, alternatively, a ‘zero’ check which tests for an acceptable number of zero interval values per day to be derived from the site’s historical metering data.

#### Check for null (no values) metering data in the metering data services database for all metering data streams.

##### The aim of this check is to ensure that there is a 100% metering data set (and substitution for any missing interval metering data is undertaken).

##### The minimum check required is to ensure that there is at least one non-null active energy or reactive energy value per interval per metering data stream.

#### Check for significant meter alarms (power failure, voltage transformer (VT) or phase failure, pulse overflow, CRC error and time tolerance).

##### A process must be in place that captures these significant meter alarms within the metering data validation process and ensures that any meter alarm occurrences are retained as part of the metering data audit trail.  Refer to clause 6.2.

##### The Metering Data Provider must provide the occurrences of these alarms to relevant Registered Participants within the metering data file that has a format compliant with the Meter Data File Format Specification.

#### Where supported by the meter(s), validation, for a given period, of interval metering data by comparison of the totalised interval energy data values (accumulation register reading)and the change in the meter cumulative registers (energy tolerance).  It is acknowledged that this check would not identify current transformer (CT) ratio changes that have occurred after initial commissioning and have not been advised to the Metering Data Provider.

## Validations to be performed for metering installations with check metering or partial check metering

### The Metering Data Provider must undertake the following validations by comparison of the metering data and check metering data for all metering installations that have associated check or partial check metering installations:

#### For metering installations where the check metering installation duplicates the metering installation accuracy level, the Metering Data Provider must validate the metering installation data streams and check metering data streams on a per interval basis.  The average of the two validated metering data sets will be used to determine the energy measurement.

#### For installations where the check metering data validation requires a comparison based on nodal balance (comparing the sum energy flow to the busbar against energy flow from the busbar), the Metering Data Provider must construct a validation algorithm within the metering data services database that will facilitate this test:

##### The Metering Data Provider must construct a validation algorithm within the metering data services database that will facilitate comparison of interval metering data for each energy flow on a per interval basis.

##### The Metering Data Provider must conduct an analysis of the historical metering data for each connection point to ascertain whether error differences in nodal balance are acceptable.

##### The Metering Data Provider should use this information to refine its validation algorithms to minimise the error difference value for each connection point, based on historical metering data.

##### The maximum error difference considered acceptable for any connection point is 1% on a per interval basis.  The Metering Data Provider should minimise this value for each connection point, based on historical data.

#### Where the check metering installation is remote from the metering installation (e.g. at the other end of a transmission line or the other side of a transformer), the validation system must employ the following functionality:

##### The Metering Data Provider must construct a validation algorithm within the metering data services database that will facilitate comparison of interval metering data from the metering installation and the check metering installation on a per interval basis with adjustment for respective transformer or line losses.

##### The Metering Data Provider must conduct an analysis of the historical metering data for each connection point to ascertain whether error differences between the metering data from the metering installation and check metering installation is acceptable.

##### The Metering Data Provider should use this information to refine its validation algorithms to minimise the error difference value for each connection point, based on historical metering data.

##### The maximum error difference considered acceptable for any connection point is 5% on a per interval basis.  The Metering Data Provider should minimise this value for each connection point, based on historical data.

#### For installation connection points where SCADA metering data is made available by AEMO for the purposes of metering data validation, the Metering Data Provider must validate the metering data by comparison of the interval metering data against the SCADA metering data as provided by AEMO in the agreed format.  The validation system must employ the following functionality:

##### The Metering Data Provider must construct a validation algorithm within the metering data services database that will facilitate comparison of interval metering data from the metering installation and the SCADA metering data on a per interval basis.

##### The Metering Data Provider must conduct an analysis of the historical metering data for each connection point to ascertain whether error differences between the interval metering data from the metering installation and the SCADA metering data is acceptable.

##### The Metering Data Provider should use this information to refine its validation algorithms to minimise the error difference value for each connection point, based on historical metering data.

##### The Metering Data Provider must construct an appropriate validation algorithm as the SCADA metering data may be derived from a different measurement point, have a different interval collection period and/or have a different base unit of measurement, (e.g. power not energy value) with allowances for a larger error of measurement.

##### The Metering Data Provider is only required to undertake validation of metering data against the SCADA metering data on the primary data channel i.e. only ‘B’ channel validation for Generators or Market Small Generation Aggregators and only ‘E’ channel validation for loads such as pumps.

# VALIDATION OF METERING DATA TYPE 4A and 5

## Application of clause 10

### Metering Data Providers accredited for the provision of metering data services for type 4A and5 metering installations must apply the requirements of:

#### Clause 10.2 for current transformer (CT) connected type 4A and 5 metering installations.

#### Clause 10.3 for whole current type 4A and 5 metering installations.

## Validations to be performed for type 4A and 5 metering installations with CTs

### The Metering Data Provider must, as a minimum, undertake the following validations on metering data within the metering data services database:

#### Check of all interval metering data against a nominated maximum value.

##### This validation is to ensure that erroneous interval metering data spikes are trapped and substituted.

##### This check may additionally be performed in the collection software.

##### A check of the maximum value of active energy.  (Maximum reactive energy checks may also be performed as an option).

##### The maximum value in 10.2.1(a)(iii) is to be initially defined by the applied current transformer (CT) ratio of the metering installation.

##### On a per installation basis, the maximum value in 10.2.1(a)(iii) may be increased to cater for situations where the metering coordinator has confirmed that the current transformer (CT) is overloaded on a short-term basis.

#### Check against a nominated minimum value or, alternatively, a ‘zero’ check which tests for an acceptable number of zero interval values per day to be derived from the site’s historical metering data.

#### Check for null (no values) metering data in the metering data services database for all metering data streams.

##### The aim of this check is to ensure that there is a 100% metering data set (and substitution for any missing interval metering data is undertaken).

##### The minimum check required is to ensure that there is at least one non-null active energy or reactive energy value per interval per metering data stream.

#### Check for significant meter alarms (power outage or power failure, VT or phase failure, pulse overflow, CRC error and time tolerance).

##### A process must be in place that captures these significant meter alarms within the metering data validation process and ensures that any meter alarm occurrences are retained as part of the metering data audit trail.  Refer to clause 6.2.

##### The Metering Data Provider must provide the occurrences of these meter alarms to relevant Registered Participants within the metering data file that has a format compliant with the Meter Data File Format Specification.

#### Where supported by the meter(s), validation, for a given period, of interval metering data by comparison of the totalised interval energy data values (accumulation register reading)and the change in the meter cumulative registers (energy tolerance).  It is acknowledged that this check would not identify current transformer (CT) ratio changes that have occurred after initial commissioning and have not been advised to the Metering Data Provider.

#### A check of the metering data for continuity and reasonability over the meter reading period.

##### Check that no gaps in the metering data exist.

##### Check that metering data for the expected period has been delivered based on the expected reading date.

## Validations to be performed for whole current type 4A and 5 metering installations

### The Metering Data Provider must, as a minimum, undertake the following validations on metering data within the metering data services database:

#### Check of all interval metering data against a nominated maximum value.

##### This validation is to ensure that erroneous interval metering data spikes are trapped and substituted.

##### This check may additionally be performed in the collection software.

##### A check of maximum value of active energy.  (Maximum reactive energy checks may also be performed as an option).  The maximum value is to be initially set to the rating of the meter.

#### Check for null (no values) metering data in the metering data services database for all metering data streams.

##### The aim of this check is to ensure that there is a 100% metering data set (and substitution for any missing metering data is undertaken).

##### The minimum check required is to ensure that there is at least one non-null active energy or reactive energy value per interval per metering data stream.

#### Check for significant meter alarms (voltage transformer (VT) or phase failure, pulse overflow, CRC error and time tolerance).  The Metering Data Provider is not required to validate the interval metering data for power outage or power failure alarms.

##### A process must be in place that captures these significant meter alarms within the metering data validation process and ensures that any meter alarm occurrences are retained as part of the metering data audit trail.  Refer to clause 6.2.

##### The Metering Data Provider must provide the occurrences of these meter alarms (inclusive of the occurrences of power outage or power failure alarms) to relevant Registered Participants within the metering data file that has a format compliant with the Meter Data File Format Specification.

#### Where supported by the meter(s), validation, for a given period, of interval metering data by comparison of the totalised interval energy data values (accumulation register reading)and the change in the meter cumulative registers (energy tolerance).

#### A check of the metering data for continuity and reasonability over the meter reading period.

##### Check that no gaps in the metering data exist.

##### Check that metering data for the expected period has been delivered based on the expected reading date.

# VALIDATION OF METERING DATA TYPE 6

## Application of clause 11

### The requirements of clause 11.2 are applicable to Metering Data Providers accredited for the provision of metering data services for metering installations type 6.

## Validations to be performed for type 6 metering installations

### The Metering Data Provider must undertake the following validations on metering data within the metering data services database:

#### Check against a nominated minimum value of metering data collected from the metering installation.

#### Check against a nominated maximum value of metering data collected from the metering installation.  This is to be applied to both the metering data collected from the metering installation and the calculated energy consumption values.

#### The current value of metering data collected from the metering installation ≥ previous value of metering data collected from the metering installation.

#### The current value of metering data collected from the metering installation is numeric and ≥ 0.

#### The current date that metering data is collected from the metering installation > the previous date that metering data was collected from the metering installation.

#### Check for null (no values) metering data in the metering data services database for all metering data streams.

##### The aim of this check is to ensure that there is a 100% metering data set (and substitution for any missing metering data is undertaken).

# VALIDATION OF METERING DATA TYPE 7

## Application of clause 12

### The requirements of clause 12.2 are applicable to Metering Data Providers accredited for provision on metering data services for type 7 metering installations.

## Validations to be performed for type 7 metering installations

### The Metering Data Provider must undertake the following validations on calculated metering data within the metering data services database:

#### Check against a nominated maximum calculated metering data value.

#### Calculated metering data value is numeric and ≥ 0.

#### Check for null (no values) calculated metering data in the metering data services database for all metering data streams.

##### The aim of this check is to ensure that there is a 100% calculated metering data set (and substitution for any missing calculated metering data has been undertaken).

#### Check the inventory tables, load tables and on/off tables with a process approved by the metering coordinator to ensure that the correct version of the tables is being used for the production of calculated metering data.

#### Check against a nominated minimum value or alternatively a ‘zero’ check which tests for an acceptable number of zero interval values per day.

#### Calculated metering data date > previous calculated metering data date.

# LOAD PROFILING – CONVERSION OF ACCUMULATED METERING DATA

## Application of clause 13

### A load profiling requirement for the conversion of accumulated metering data into trading intervals applies to the jurisdictions of Victoria, New South Wales, Queensland, South Australia, Australian Capital Territory, and Tasmania.

## Requirement for load profiling

### Load profiling is required in order to determine trading interval metering data, for wholesale market settlement purposes, for type 6 metering installations.

### The requirements for load profiling in Victoria, Australian Capital Territory, and Tasmania are to:

#### Determine an estimate of the average load profile for a profile area over a given period of time (Profile Preparation Service); and

#### Allocate that load profile to end-use customers in that profile area (Basic Meter Profiler).

### The requirements for load profiling in NSW and Queensland are to:

#### Determine an estimate of the average load profile for first-tier and second-tier controlled loads for a profile area over a given period of time (Profile Preparation Service – Controlled Load Profile);

#### Allocate that profile to both first-tier and second-tier controlled loads metering data (Basic Meter Profiler – Controlled Load Profile);

#### Determine an estimate of the average load profile of the remaining first-tier and second-tier loads for a profile area (that is, excluding the first-tier and second-tier controlled loads) over a given period of time (Profile Preparation Service – Net System Load Profile); and

#### Allocate that load profile to second-tier non-controlled loads in that profile area (Basic Meter Profiler – Net System Load Profile).

### The requirements for load profiling in South Australia are to:

#### Determine an estimate of the average load profile for first-tier and second-tier controlled loads for a profile area over a given period of time (Profile Preparation Service – Controlled Load Profile);

#### Allocate that profile to second-tier controlled loads in that profile area (Basic Meter Profiler – Controlled Load Profile);

#### Determine an estimate of the average load profile of the remaining first-tier and second-tier loads for a profile area (that is, excluding the first-tier and second-tier controlled loads) over a given period of time (Profile Preparation Service – Net System Load Profile); and

#### Allocate that load profile to second-tier non-controlled loads in that profile area (Basic Meter Profiler – Net System Load Profile).

## Controlled load profile

### Clause 13.3 applies to the Profile Preparation Service – Controlled Load. The requirements for the Profile Preparation Service - Controlled Load and the actions to be undertaken in each applicable jurisdiction are as specified in the following sub-clauses.

### Profile Preparation Service – Controlled Load does not apply to the jurisdictions of the Australian Capital Territory, Tasmania and Victoria.

### Profile Preparation Service - Controlled Load is applied in the jurisdiction of New South Wales as follows:

#### In accordance with clause 3.10.2 of Metrology Procedure: Part A, controlled load profile(s) (CLP) for each profile area must be prepared by AEMO using interval metering data from a sample (or samples) of controlled load interval meters in accordance with clause 13.3.7 and sub-clauses 13.3.3 (b) and (c).

#### The sample meters, which will be installed by the NSW Local Network Service Providers, must be a type 4 metering installation.

#### Two NMIs must be allocated to each sample meter.

##### One NMI must be used for the interval metering data from the sample meter that is used to prepare the CLP in accordance with this clause 13.3.3; and

##### Where the metering installation that has a sample meter is second-tier, a second NMI must be used to transfer the accumulated metering data to which the CLP is applied in accordance with clause 13.3.6.

#### The requirements for one controlled load profile are:

##### There shall be at least one CLP in each profile area.

##### A Local Network Service Provider may choose to introduce a second CLP in its profile area.

##### If the Local Network Service Provider chooses not to introduce a second CLP, one CLP must be calculated for all controlled loads in a profile area, which is based on a sample of controlled load interval meters.

##### For each half hourly trading interval, the CLP must be calculated by profile area as follows:

##### CLP for a profile area for a trading interval j

##### = (sample meter load in trading interval j)n  (wf) n

##### where:
N = represents the set of sample NMIs in the profile area
wf = is the weighting factor associated with the NMIs.

#### The requirements for two controlled load profiles are:

##### If the Local Network Service Provider chooses to introduce a second CLP, it must provide a written notice advising of the commencement date of the second CLP to:

AEMO, and

All Retailers.

##### The commencement date must be at least six months after the date of the notice.

##### From the commencement date, two CLPs must be calculated for the profile area:

One for controlled loads in the profile area based on a sample of controlled load interval meters on the controlled load 1 network tariff; and

One for controlled loads in the profile area based on a sample of controlled load interval meters on the controlled load 2 network tariff.

#### For each half hourly trading interval, the CLPs must be calculated by profile area as follows:

##### CLP for loads on the controlled load 1 network tariff

##### where:
N = represents the set of sample NMIs on the controlled load 1 network tariff, in the profile area.
wf = weighting factor associated with the NMIs.

##### CLP for loads on the controlled load 2 network tariff

##### where:
M =represents the set of sample NMIs on the controlled load 2 network tariff, in the profile area
wf = weighting factor associated with the NMIs.

### The requirements for the Profile Preparation Service - Controlled Load in Queensland are:

#### In accordance with clause 3.10.2 of Metrology Procedure: Part A, controlled load profiles (CLPs) for the profile area must be prepared by AEMO using interval metering data from a sample (or samples) of controlled load interval meters in accordance with clause 13.3.7 and sub-clauses 13.3.4 (b) and (c).

#### The sample meters, which will be installed by the Local Network Service Provider, must be type 4 metering installations.

#### Two NMIs may need to be allocated to each sample meter.

##### One NMI must be used for the interval metering data from the sample meter that is used to prepare the CLP in accordance with this clause 13.3.4; and

##### Where the metering installation that has a sample meter is second-tier, a second NMI must be used to transfer the accumulated metering data to which the CLP is applied in accordance with clause 13.3.6.

#### Two CLPs must be calculated for the profile area:

##### One for controlled loads in the profile area based on a sample of controlled load interval meters on the controlled load 1 network tariff; and

##### One for controlled loads in the profile area based on a sample of controlled load interval meters on the controlled load 2 network tariff.

#### For each half hourly trading interval, the CLPs must be calculated by profile area as follows:

##### CLP for loads on the controlled load 1 network tariff

##### where:
N = represents the set of sample NMIs on the controlled load 1 network tariff, in the profile area.
wf = weighting factor associated with the NMIs.

##### CLP for loads on the controlled load 2 network tariff

##### where:
M =represents the set of sample NMIs on the controlled load 2 network tariff, in the profile area
wf = weighting factor associated with the NMIs.

### The requirements for the Profile Preparation Service - Controlled Load in South Australia are:

#### In accordance with clause 3.10.2 of Metrology Procedure: Part A, a single controlled load profile (CLP) for all controlled loads for each profile area must be prepared by AEMO using interval metering data from a sample of controlled load interval meters in accordance with clause 13.3.7 and clauses 13.3.5 (b) and (c).

#### The sample meters, which will be installed by the South Australian Local Network Service Provider, must be a type 4 metering installation.

#### Two NMIs may need to be allocated to each sample meter.

##### One NMI must be used for the interval metering data from the sample meter that is used to prepare the CLP in accordance with this clause 13.3.5; and

##### Where the metering installation that has a sample meter is second-tier, then a second NMI must be used to transfer the accumulated metering data to which the CLP is applied in accordance with clause 13.3.6.

#### For each half hourly trading interval, the controlled load profile must be calculated by profile area as follows:

##### CLP for a profile area for a trading interval j

##### =(sample meter load in trading interval j)n  (wf) n

#### where:
N = represents the set of sample NMIs in the profile area
wf = is the weighting factor associated with the NMIs.

### For the Basic Meter Profiler – Controlled Load, actions are to be undertaken in each jurisdiction as specified in the following sub-clauses:

#### The requirements for Basic Meter Profiler – Controlled Load in New South Wales and Queensland are:

##### In accordance with clause 3.10.2 of Metrology Procedure: Part A, AEMO must apply the appropriate CLP, for the profile area to which the NMI is connected, to the accumulated metering data for all first-tier and second-tier controlled loads, in order to obtain trading interval metering data.

##### The requirements for one controlled load profile are:

###### For NMIs in a profile area with one CLP, the CLP must be applied as follows:

###### Half hourly metering data for trading interval j for a NMI data stream

###### where:
CLPj = the calculated controlled load profile energy for trading interval j
 = the sum of controlled load profile energy between the start date and the end date and;

###### ‘start date’ and ‘end date’ have the meanings given in clause 13.5.

##### The requirements for two controlled load profiles are:

###### In a profile area where the Local Network Service Provider has chosen to introduce a second CLP, the CLPs shall be applied as follows:

Loads on the controlled load 1 network tariff (CLP1) must be applied to the accumulated metering data for all first tier and second-tier controlled loads, which are on the controlled load 1 network tariff:

- Half hourly metering data for trading interval j for a NMI data stream on the controlled load 1 network tariff

###### where:
 = the calculated controlled loadpProfile energy for trading interval j
 = the sum of controlled load profile energy between the start date and the end date and;

###### ‘start date’ and ‘end date’ have the meanings given in clause 13.5.

Loads on the controlled load 2 network tariff (CLP2) must be applied to the accumulated metering data for all first-tier and second-tier controlled loads, which are on the controlled load 2 network tariff:

- Half hourly metering data for trading interval j for a NMI data stream on the controlled load 2 network tariff

###### where:

###### = the calculated controlled load profile energy for trading interval j
 = the sum of controlled load profile energy between the start date and the end date and;

- ‘start date’ and ‘end date’ have the meanings given in clause 13.5.

##### The resulting half hourly metering data produced by applying the controlled load profile is at the NMI data stream level.  The total of these data streams is used in the calculation of the Net System Load Profile as explained in clause 13.4.2.

#### The requirements for Basic Meter Profiler – Controlled Load in South Australia are:

##### In accordance with clause 3.10.2 of Metrology Procedure: Part A, AEMO must apply the CLP, for the profile area to which the NMI is connected, to the accumulated metering data for all first-tier and second-tier controlled loads, in order to obtain trading interval metering data.

##### The accumulated metering data for first-tier loads is represented by a weekly load scaling factor that is transferred to AEMO in accordance with clause 3.10.5 of Metrology Procedure: Part A.

##### The profile must be applied as follows:

##### Half hourly metering data for trading interval j for a NMI data stream

###### where:
CLPj = the calculated controlled load profile energy for trading interval j
 = the sum of controlled load profile energy between the start date and the end date and;

##### ‘start date’ and ‘end date’ have the meanings given in clause 13.5.

##### If the accumulated metering data is based on an actual meter reading:

###### Start date = 00:00 on the day of the previous meter reading.

###### End date = the end of the trading interval commencing at 23:30 on the day prior to the current meter reading date.

##### If the accumulated metering data is a forward estimate:

###### Start date = 00:00 on the first day of the billing period, or 00:00 on the previous meter reading date (whether actual or forward estimate), or 00:00 on the first day that the load becomes second-tier, whichever is the later.

###### End date = the end of the trading interval commencing at 23:30 on the last day of the billing period, or the end of the trading interval commencing at 23:30 on the forward estimate meter reading date, whichever is the earlier.

##### The resulting half hourly metering data produced by applying the controlled load profile is at the NMI data stream level.  The total of these data streams is used in the calculation of the Net System Load Profile as explained in clause 13.4.2.

### Sample Meters:

#### Where metering equipment to sample controlled loads is installed, the metering coordinator must ensure that:

##### At least 200 controlled load sample meters are installed for the purposes of calculating each controlled load profile within a profile area unless otherwise agreed by AEMO; and

##### The method it adopts for selecting sample sites is statistically sound.

#### This sub-clause (b) relating to sample size and selection only applies to the jurisdiction of South Australia.

##### The Local Network Service Provider must ensure that at least 200 controlled load sample meters are installed for the purposes of calculating the controlled load profile.

##### The method of selecting sample meter sites adopted by the Local Network Service Provider must be approved by AEMO.

##### The Local Network Service Provider must use reasonable endeavours to ensure that sample meter sites:

###### Are occupied sites;

###### Have historical annual energy consumption with a lower range exceeding 1,000kWh and an upper range not exceeding two standard deviations of the mean controlled load annual consumption;

###### Subject to sub-clause (4), must be randomly selected such that each site which meets the other criteria has an equal chance of being included in the sample; and

###### New sample metering sites are selected to maintain a sample distribution that is representative of the controlled load in accordance with clause 13.3.7(b)(ii).

##### In the event that an existing sample meter site becomes inappropriate for a sample meter, the sample meter must be removed or relocated to an appropriate site selected in accordance with clause 13.3.7 (b) (iii).

##### The Local Network Service Provider must ensure that a meter which is a sample interval meter installed for the purposes of calculating the controlled load profile is not removed without the consent of AEMO.

#### The weighting factor assigned to each sample meter NMI in the calculation of the controlled load profile should be proportional to 1/n, where n is the number of sample meters contributing to the calculation of the controlled load profile in the respective Local Network Service Provider area.

#### Weighting Factor

#### where :

#### sf = scaling factor (for South Australia = 200)

#### dlf = distribution loss factor applicable to the sample meter site

#### n = number of sample meters used in the calculation of the controlled load profile

#### This sub-clause (d) relating to Weekly Load Scaling Factor only applies to the jurisdiction of South Australia.

##### AEMO must approve the calculation of the load scaling factor for the controlled load profile at a frequency which is at least once every six  months.

##### For each week, the Local Network Service Provider must calculate the load scaling factor as follows:

##### where:

##### Weekly Load Scaling Factor (MWh) = a value calculated to be the total energy consumed for a one week period by accumulation metered controlled load end-use customers that are first-tier loads.

##### X = the estimated proportion of accumulation metered controlled load end-use customers that are first tier loads determined for the six month period beginning 1 July and 1 January respectively.

##### Annual CL Energy (MWh) = the annual controlled load energy of all end-use customers for the whole calendar year preceding the regulatory year t.

##### Annual Sample Meter CL Energy (MWh) = total annual sample meter controlled load energy for the whole calendar preceding regulatory year t.

##### Weekly Sample Energy (MWh) = the value representing the actual energy consumption recorded by the sample meters in a one week period.

##### The Local Network Service Provider must send the load scaling factor to AEMO as an active first-tier load accumulation metered NMI data stream, with the actual reading date as the last day in the settlement week, at a frequency which is at least once per week.

## Net system load profile

### Clause 13.4 applies to the jurisdictions of Victoria, New South Wales, South Australia, Queensland, the Australian Capital Territory, and Tasmania.

### The requirements for the Profile Preparation Service – Net System Load Profile are:

#### In accordance with clause 3.10.2 of Metrology Procedure: Part A, the form of profiling that AEMO must use for the metering installations to which the metrology procedure applies, excluding metering installations for controlled loads where applicable to a jurisdiction, is the Net System Load Profile (NSLP).

#### The NSLP must be calculated by profile area as follows:

#### NSLP for a profile area for a trading interval

-

where

MLF = marginal loss factor applicable for the NMI that is stored in MSATS

DLF = distribution loss factor applicable for the NMI that is stored in MSATS

i = Each TNI with energy inflows to profile area

m = Each Embedded Generator with energy generated within profile area

s = Half hourly loads in profile area, which include:

•	interval metered second-tier loads;

•	interval metered first-tier loads in accordance with clause 3.6.4 of Metrology Procedure: Part A;

•	unmetered first-tier and second-tier loads;

•	interval metered market loads that are not first-tier or second-tier loads;

•	where applicable to a participating jurisdiction, interval metering data for first-tier and second-tier controlled loads; and

•	but exclude child interval metered loads in an embedded network.

### The requirements for the Basic Meter Profiler – Net System Load Profile are:

#### In accordance with clause 3.10.2 of Metrology Procedure: Part A, AEMO must apply the NSLP, for the profile area to which the NMI is connected, to the metering data for type 6 metering installations, in order to obtain interval metering data.

#### The profile must be applied as follows:

#### Half-hourly metering data for trading interval j for NMI data stream

where the accumulated metering data is based on an actual meter reading, then

start date = 00:00 on the day of the previous meter reading, and

end date = the end of the trading interval commencing at 23:30 on the day prior to the current meter reading date.

and where the accumulated metering data is a forward estimate, then

start date = 00:00 on the first day of the billing period, or 00:00 on the day of the previous meter reading date (whether actual or forward estimate), or 00:00 on the first day that the load becomes second-tier, whichever is the later, and

end date = the end of the trading interval commencing at 23:30 on the last day of the billing period, or the end of the trading interval commencing at 23:30 on the forward estimate meter reading date, whichever is the earlier.

#### The resulting half hourly metering data produced by applying the NSLP is at the NMI data stream level.

## Start dates and end dates

### For the purpose of clause 13.3.6:

##### If the accumulated metering data is based on an actual meter reading:

###### The start date is 00.00 on the day of the previous meter reading; and

##### The end date is the end of the trading interval commencing at 23.30 on the day prior to the current meter reading date; and

##### If the accumulated metering data is a forward estimate:

###### The start date is the later of:
(A) 00.00 on the first day of the billing period;
(B) 00.00 on the previous meter reading date (whether actual or forward estimate; and
(C) 00.00 on the first day on which the load becomes second-tier:

###### The end date is the earlier of:
(A) the end of the trading interval commencing at 23.30 on the last day of the billing period; and
(B) the end of the trading interval commencing at 23.30 on the forward estimate meter reading date.

# UNMETERED LOADS – DETERMINATION OF metering data

## Application of clause 14

### The requirements of clause 14 apply to the jurisdictions of Victoria, New South Wales, Queensland, South Australia, Tasmania and Australian Capital Territory.

## Requirement to produce calculated metering data

### Agreed market loads

#### In accordance with S7.6 and S7.2 of the Rules, half-hourly trading interval data is required to be calculated by NMI data stream for those loads that have been classified as type 7 metering installations in accordance with S7.2 of the Rules that, subject to clause 14.2.1 (d), are able under relevant legislation or regulation to be classified as market loads as agreed between a Minister and AEMO.

#### AEMO will publish a list of the loads that have been classified, in accordance with clause 14.2.1 (a), as market loads and any amendments to these loads.

#### Loads that have been classified, in accordance with clause 14.2.1 (a), as market loads may be either a “controlled unmetered load” (for which the market load is controlled) or “other unmetered load” (for which the market load is not controlled).

#### If there is a load table under clause 14.2.4, an inventory table under clause 14.3.3 or clause 14.4.3 and for controlled unmetered loads an on/off table under clause 14.3.4, then a type 7 metering installation may be classified as a market load in accordance with clause 14.2.1 (a).

### Application to device types

### The agreed market load that is published by AEMO will be generic in nature (for example, street lighting).  For each agreed market load there may be one or more device types that are listed in the load table developed in accordance with clause 14.2.4.

### Application of NMI

#### Metering data for an unmetered load is calculated by NMI data stream.  A NMI is assigned for each unique combination of:

##### Financially responsible Market Participant;

##### End-use customer;

##### Local Network Service Provider;

##### Transmission Node Identity; and

##### Distribution loss factor.

#### The unmetered load NMI may contain different agreed market loads and/or different device types, but they must have the same financially responsible Market Participant, end-use customer, Local Network Service Provider, TNI and distribution loss factor.

#### Where permitted by the Rules or guidelines issued by AEMO, an unmetered load may be included in the NMI for a related metered load, where the number of devices is small, for example watchman lights, the energy consumption of those devices is immaterial relative to the total energy consumption for that NMI, and the financially responsible Market Participant, end-use customer, Local Network Service Provider, TNI and distribution loss factor are the same.

### Load Table

#### The load table must set out:

##### The device load (in watts) for controlled unmetered loads for use in calculating interval metering data for each device type in accordance with clause 14.3.  The load per device type must be the wattage of the device and associated control gear; and

##### The annual energy consumption for other unmetered loads by device type in accordance with clause 14.4.1.  The annual energy consumption is used to calculate the calculated device wattage (in watts) which is used to calculate the interval metering data for each device type as follows:

### Where i = device type i.

#### AEMO must develop the initial load table in accordance with the Rules consultation procedures.

#### The initial load table must be published by AEMO.  Changes to the load table must be published by AEMO.  AEMO should remove redundant device types from the published load table.

#### AEMO must consider and publish proposals from interested parties to add device types to the load table.  AEMO must determine whether to add the new device type and its corresponding device load or annual energy consumption to the load table taking into account the views of other interested parties.  Agreement of device load to be added to the NEM Load Table does not replace any obligation for an interested party to obtain appropriate approvals related to the performance and acceptance of use of the device.

#### New device types must be included in the load table prior to installation of the device.

#### AEMO must develop and publish a guideline for the determination of a load table device load or annual energy consumption.  The guideline must be prepared, revised and published by AEMO in accordance with the Rules consultation procedures.

#### Proposals to add a new load value for an unmetered device type to the load table must be accompanied by a relevant unique description of the device and evidence of the device load or energy consumption whichever the case may be.  Wherever possible, the device load should be determined from measurement tests conducted by a NATA accredited laboratory or overseas equivalent.

## Controlled unmetered loads

### This clause 14.3 is applicable to all agreed “controlled unmetered loads”.

### Metering Data calculation

#### The metering coordinator must ensure that the interval metering data for controlled unmetered loads, which have been classified as a type 7 metering installation, are calculated in accordance with the following algorithm:

#### Half-hourly metering data for trading interval j for NMI (in watt hours)

where;

i = device type

j = trading interval

k = proportion of device attributable to that NMI

Trading interval is period in minutes

Device wattage is determined from the load table.

Device count is determined from the inventory table.

Period load is switched on is determined from the on/off table.

### Inventory Table

#### For each NMI, a separate inventory table is required that identifies each device type which forms part of the NMI load and for each device type lists:

##### The device type;

##### The form of on/off control – photoelectric cell control, timer control, ripple control or other control;

##### If timer control or ripple control, the on/off times for the controlling device;

##### If other control, the on/off times;

##### If a device is shared with another NMI, the proportion of load that is agreed by affected Registered Participants to be attributable to that NMI (k).  Each k factor will be less than 1.  The sum of the k factors for a shared device across each respective NMI must be equal to 1;

##### If a device is not shared with another NMI, the k factor must be equal to 1.

##### Number of such devices installed;

##### Effective start date – the first day on which that record in the inventory table is to be included in the calculation of metering data for that NMI;

##### Effective end date – the last day on which that record in the inventory table is to be included in the calculation of metering data for that NMI; and

##### Last change date – the date that record in the inventory table was most recently created or modified.

#### Each device in the inventory table is a unique combination of physical hardware, time control classification and shared portion.  For example, if a device is shared with another NMI, the individual portions of the device(s) must be included in the inventory table as a separate device type on each NMI.

#### Each metering coordinator must develop the initial inventory table for the NMIs for which it is responsible.  The initial inventory table must be agreed by the affected Registered Participants, AEMO and the relevant end-use customer.

#### Each metering coordinator must use reasonable endeavours to update the inventory table, for the NMIs for which it is responsible, on at least a monthly basis for any additions, deletions and modifications to ensure that the accuracy requirements in clause 3.7.7 of Metrology Procedure: Part A are met.  Such additions, deletions or modifications to the inventory table may only be made on a retrospective basis where:

##### Agreed by the metering coordinator and the affected Registered Participants; or

##### Necessary to comply with clause 7.9.5 of the Rules.

#### The metering coordinator must communicate any material changes to the inventory table to the affected Registered Participants.

#### The metering coordinator must provide the inventory table to relevant Registered Participants when requested.

### On/Off table

#### The form of on/off control may be:

##### Photoelectric cell control;

##### Timer control, or ripple control; or

##### Other control.

### Photoelectric cell control

#### If the on/off times for a device is controlled by a photoelectric cell, then

##### On time = sunset time + ON delay

##### Off time = sunrise time + OFF delay.

#### The ON delay and OFF delay are set out in clause 14.5.

#### The metering coordinator must ensure that the appropriate sunset times and sunrise times are obtained from the Australian Government Geoscience website (www.ga.gov.au/geodesy/astro/sunrise.jsp), based on the longitude and latitude of the relevant town as specified below, and Eastern Standard Time:

| #### Jurisdiction | #### LNSP | #### Town | #### Latitude | #### Longitude |
| --- | --- | --- | --- | --- |
| #### Victoria | #### CitiPower | #### Melbourne | #### 37 deg 49 min S | #### 144 deg 58 min E |
| #### Victoria | #### Jemena Electricity Networks | #### Essendon | #### 37 deg 44 min S | #### 144 deg 54 min E |
| #### Victoria | #### Powercor | #### Ballarat | #### 37 deg 30 min S | #### 143 deg 47 min E |
| #### Victoria | #### AusNet Services | #### Morwell | #### 38 deg 13 min S | #### 146 deg 25 min E |
| #### Victoria | #### United Energy | #### Dandenong | #### 38 deg 01 min S | #### 145 deg 12 min E |
| #### NSW | #### Ausgrid | #### Sydney | #### 33 deg 52 min S | #### 151 deg 12 min E |
| #### NSW | #### Endeavour Energy | #### Cecil Park | #### 33 deg 52 min S | #### 150 deg 50 min E |
| #### NSW | #### Essential Energy | #### Armidale | #### 30 deg 31 min S | #### 151 deg 40 min E |
| #### NSW | #### Essential Energy | #### Broken Hill | #### 31 deg 57 min S | #### 141 deg 27 min E |
| #### NSW | #### Essential Energy | #### Dubbo | #### 32 deg 15 min S | #### 148 deg 36 min E |
| #### NSW | #### Essential Energy | #### Wagga Wagga | #### 35 deg 06 min S | #### 147 deg 22 min E |
| #### SA | #### SA Power Networks | #### Adelaide | #### 34 deg 55 min S | #### 138 deg 35 min E |
| #### ACT | #### ActewAGL | #### Canberra | #### 35 deg 20 min S | #### 149 deg 10 min E |
| #### Queensland | #### Energex | #### Brisbane | #### 27 deg 28 min S | #### 153 deg 01 min E |
| #### Queensland | #### Ergon Energy | #### Townsville | #### 19 deg 15 min S | #### 146 deg 48 min E |
| #### Queensland | #### Ergon Energy | #### Toowoomba | #### 27 deg 33 min S | #### 151 deg 57 min E |
| #### Tasmania | #### Aurora Distribution | #### Ross | #### 42 deg 01 min S | #### 147 deg 29 min E |

#### The metering coordinator must ensure that the period that the load is switched on during a trading interval is calculated as follows:

| Trading interval | Period load is switched on |
| --- | --- |
| For the trading intervals commencing after sunset and finishing prior to sunrise | Period load is switched on = 1 |
| For the trading intervals commencing after sunrise and finishing prior to sunset | Period load is switched on = 0 |
| For the trading interval during which the sunset occurs |  |
| For the trading interval during which the sunrise occurs |  |

#### Should testing on the operation of photoelectric cells by an independent party, agreed to by the metering coordinator, affected Registered Participants, AEMO and relevant end-use customer, indicate that the on/off times for a device controlled by a photoelectric cell are influenced materially and consistently by other variables, then AEMO shall revise the metrology procedure accordingly.

### Timer control

#### If the on/off times for a device is controlled by a timer or ripple injection system, then

##### On time = ON time set on timer or ripple injection system

##### Off time = OFF time set on timer or ripple injection system.

#### The metering coordinator must ensure that the period that the load is switched on during a trading interval is calculated as follows:

| Trading interval | Period load is switched on |
| --- | --- |
| For the trading intervals commencing after on time and finishing prior to off time | Period load is switched on = 1 |
| For the trading intervals commencing after off time and finishing prior to on time | Period load is switched on = 0 |
| For the trading interval during which the on time occurs |  |
| For the trading interval during which the off time occurs |  |

### Other control

#### Where the on/off times for a device are not in accordance with clause 14.3.5 or clause 14.3.6, then the following alternative forms of control may be used.

##### On time = sunset time + ON delay or ON time set on timer or ripple injection system.

##### Off time = sunrise time + OFF delay or OFF time set on timer or ripple injection system or a fixed duration after ON time.

#### Where sunrise or sunset times are used, the time is determined in accordance with clause 14.3.5(b).

#### The metering coordinator must ensure that the period that the load is switched on during a trading interval is calculated as follows:

| Trading interval | Period load is switched on |
| --- | --- |
| For the trading intervals commencing after on time and finishing prior to off time | Period load is switched on = 1 |
| For the trading intervals commencing after off time and finishing prior to on time | Period load is switched on = 0 |
| For the trading interval during which the on time occurs |  |
| For the trading interval during which the off time occurs |  |

## Other unmetered loads

### This clause 14.4 is applicable to all agreed “other unmetered loads”.  Other unmetered loads” do not have a constant load and therefore the energy is calculated based on an annual energy consumption determined by AEMO in accordance with clause 14.2.4.

### Energy calculation

#### The metering coordinator must ensure that the interval metering data for other unmetered loads, which have been classified as a type 7 metering installation, is calculated in accordance with the following algorithm.

#### Half-hourly metering data for trading interval j for NMI (in watt hours)

where;

i = device type

j = trading interval

k = proportion of device attributable to that NMI

Trading interval is period in minutes

Device wattage is determined from the load table.

Device count is determined from the inventory table.

Period load is switched on is determined from the on/off table.

### Inventory Table

#### For each NMI, a separate inventory table is required that identifies each device type which forms part of the NMI load and for each device type lists:

##### The device type;

##### The form of on/off control (24 hours per day);

##### If a device is shared with another NMI, the proportion of load that is agreed by affected Registered Participants to be attributable to that NMI (k).  Each k factor will be less than 1.  The sum of the k factors for a shared device across each respective NMI must be equal to 1;

##### If a device is not shared with another NMI, the k factor must be equal to 1.

##### Number of such devices installed;

##### Effective start date – the first day on which that record in the inventory table is to be included in the calculation of metering data for that NMI;

##### Effective end date – the last day on which that record in the inventory table is to be included in the calculation of metering data for that NMI; and

##### Last change date – the date that record in the inventory table was most recently created or modified.

#### Each device in the inventory table is a unique combination of physical hardware, time control classification and shared portion.  For example, if a device is shared with another NMI, the individual portions of the device(s) shall be included in the inventory table as a separate device type on each NMI.

#### Each metering coordinator must develop the initial inventory table for the NMIs for which it is responsible.  The initial inventory table must be agreed by the affected Registered Participants, AEMO and the relevant end-use customer.

#### Each metering coordinator must use reasonable endeavours to update the inventory table, for the NMIs for which it is responsible, on at least a monthly basis for any additions, deletions and modifications to ensure that the accuracy requirements in clause 3.7.7 of Metrology Procedure: Part A are met.  Such additions, deletions or modifications to the inventory table may only be made on a retrospective basis where:

##### Agreed by the metering coordinator and the affected Registered Participants; or

##### Necessary to comply with clause 7.9.5 of the Rules.

#### The metering coordinator must communicate any material changes to the inventory table to the affected Registered Participants.

#### The metering coordinator must provide the inventory table to relevant Registered Participants when requested.

### On/off Table

#### Other unmetered loads are assumed to operate 24 hours per day.  For each trading interval:

#### Period load is switched on = 1.

## ON delay and OFF delay

### This clause 14.5 applies only to the jurisdiction of the Australian Capital Territory.  For the jurisdictions of Victoria, New South Wales, Queensland, Tasmania and South Australia the ON and OFF delays are zero.

#### The metering coordinator must use the ON delay and OFF delay for each day of the year as provided in the following tables, when determining the on time and off time of photoelectric cells in accordance with clause 14.3.5.

January

| Day | ON delay (minutes) | OFF delay (minutes) | Day | ON delay (minutes) | OFF delay (minutes) |
| --- | --- | --- | --- | --- | --- |
| January 1 January 2 January 3 January 4 January 5 January 6 January 7 January 8 January 9 January 10 January 11 January 12 January 13 January 14 January 15 | 20 19 19 19 19 19 19 19 19 19 19 19 19 19 19 | 21 20 20 20 20 20 20 20 20 20 20 20 20 20 20 | January 16 January 17 January 18 January 19 January 20 January 21 January 22 January 23 January 24 January 25 January 26 January 27 January 28 January 29 January 30 January 31 | 19 19 19 19 19 19 19 19 19 18 18 18 18 18 18 18 | 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 |

February

| Day | ON delay (minutes) | OFF delay (minutes) | Day | ON delay (minutes) | OFF delay (minutes) |
| --- | --- | --- | --- | --- | --- |
| February 1 February 2 February 3 February 4 February 5 February 6 February 7 February 8 February 9 February 10 February 11 February 12 February 13 February 14 | 18 18 18 18 18 18 18 18 18 18 18 18 18 18 | 20 20 20 20 20 20 20 20 20 20 20 20 20 20 | February 15 February 16 February 17 February 18 February 19 February 20 February 21 February 22 February 23 February 24 February 25 February 26 February 27 February 28 February 29 | 18 17 17 17 17 17 17 17 17 17 17 17 17 17 17 | 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 |

March

| Day | ON delay (minutes) | OFF delay (minutes) | Day | ON delay (minutes) | OFF delay (minutes) |
| --- | --- | --- | --- | --- | --- |
| March 1 March 2 March 3 March 4 March 5 March 6 March 7 March 8 March 9 March 10 March 11 March 12 March 13 March 14 March 15 | 17 17 17 17 17 17 17 17 17 17 16 16 16 16 16 | 20 20 20 19 19 19 19 19 19 19 19 19 19 19 19 | March 16 March 17 March 18 March 19 March 20 March 21 March 22 March 23 March 24 March 25 March 26 March 27 March 28 March 29 March 30 March 31 | 16 16 16 16 16 16 16 16 16 16 16 16 16 16 16 16 | 19 19 19 19 19 19 19 19 19 19 19 19 19 19 19 19 |

April

| Day | ON delay (minutes) | OFF delay (minutes) | Day | ON delay (minutes) | OFF delay (minutes) |
| --- | --- | --- | --- | --- | --- |
| April 1 April 2 April 3 April 4 April 5 April 6 April 7 April 8 April 9 April 10 April 11 April 12 April 13 April 14 April 15 | 16 16 15 15 15 15 15 15 15 15 15 15 15 15 15 | 19 19 19 19 19 19 19 19 19 19 19 19 19 19 19 | April 16 April 17 April 18 April 19 April 20 April 21 April 22 April 23 April 24 April 25 April 26 April 27 April 28 April 29 April 30 | 15 15 15 15 15 15 15 15 15 15 14 14 14 14 14 | 19 19 19 19 19 19 19 19 19 19 19 19 19 19 19 |

May

| Day | ON delay (minutes) | OFF delay (minutes) | Day | ON delay (minutes) | OFF delay (minutes) |
| --- | --- | --- | --- | --- | --- |
| May 1 May 2 May 3 May 4 May 5 May 6 May 7 May 8 May 9 May 10 May 11 May 12 May 13 May 14 May 15 | 14 14 14 14 14 14 14 14 14 14 14 14 14 14 14 | 19 19 19 18 18 18 18 18 18 18 18 18 18 18 18 | May 16 May 17 May 18 May 19 May 20 May 21 May 22 May 23 May 24 May 25 May 26 May 27 May 28 May 29 May 30 May 31 | 14 14 14 13 13 13 13 13 13 13 13 13 13 13 13 13 | 18 18 18 18 18 18 18 18 18 18 18 18 18 18 18 18 |

June

| Day | ON delay (minutes) | OFF delay (minutes) | Day | ON delay (minutes) | OFF delay (minutes) |
| --- | --- | --- | --- | --- | --- |
| June 1 June 2 June 3 June 4 June 5 June 6 June 7 June 8 June 9 June 10 June 11 June 12 June 13 June 14 June 15 | 13 13 13 13 13 13 13 13 13 12 12 12 12 12 12 | 18 18 18 18 18 18 18 18 18 18 18 18 18 18 18 | June 16 June 17 June 18 June 19 June 20 June 21 June 22 June 23 June 24 June 25 June 26 June 27 June 28 June 29 June 30 | 12 12 12 12 12 12 12 12 12 12 12 12 12 12 12 | 18 18 18 18 18 18 18 18 18 18 18 18 18 18 18 |

July

| Day | ON delay (minutes) | OFF delay (minutes) | Day | ON delay (minutes) | OFF delay (minutes) |
| --- | --- | --- | --- | --- | --- |
| July 1 July 2 July 3 July 4 July 5 July 6 July 7 July 8 July 9 July 10 July 11 July 12 July 13 July 14 July 15 | 12 12 12 12 12 12 12 12 12 12 12 12 12 12 12 | 18 18 18 18 18 18 18 18 18 18 18 18 18 18 18 | July 16 July 17 July 18 July 19 July 20 July 21 July 22 July 23 July 24 July 25 July 26 July 27 July 28 July 29 July 30 July 31 | 12 12 12 12 12 13 13 13 13 13 13 13 13 13 13 13 | 18 18 18 18 18 18 18 18 18 18 18 18 18 18 18 18 |

August

| Day | ON delay (minutes) | OFF delay (minutes) | Day | ON delay (minutes) | OFF delay (minutes) |
| --- | --- | --- | --- | --- | --- |
| August 1 August 2 August 3 August 4 August 5 August 6 August 7 August 8 August 9 August 10 August 11 August 12 August 13 August 14 August 15 | 13 13 13 13 13 13 13 13 13 13 13 14 14 14 14 | 18 18 18 18 18 18 18 18 18 18 18 18 18 18 18 | August 16 August 17 August 18 August 19 August 20 August 21 August 22 August 23 August 24 August 25 August 26 August 27 August 28 August 29 August 30 August 31 | 14 14 14 14 14 14 14 14 14 14 14 14 14 14 14 14 | 18 18 18 18 18 18 18 18 18 18 18 19 19 19 19 19 |

September

| Day | ON delay (minutes) | OFF delay (minutes) | Day | ON delay (minutes) | OFF delay (minutes) |
| --- | --- | --- | --- | --- | --- |
| September 1 September 2 September 3 September 4 September 5 September 6 September 7 September 8 September 9 September 10 September 11 September 12 September 13 September 14 September 15 | 14 14 14 15 15 15 15 15 15 15 15 15 15 15 15 | 19 19 19 19 19 19 19 19 19 19 19 19 19 19 19 | September 16 September 17 September 18 September 19 September 20 September 21 September 22 September 23 September 24 September 25 September 26 September 27 September 28 September 29 September 30 | 15 15 15 15 15 15 15 15 15 15 15 16 16 16 16 | 19 19 19 19 19 19 19 19 19 19 19 19 19 19 19 |

October

| Day | ON delay (minutes) | OFF delay (minutes) | Day | ON delay (minutes) | OFF delay (minutes) |
| --- | --- | --- | --- | --- | --- |
| October 1 October 2 October 3 October 4 October 5 October 6 October 7 October 8 October 9 October 10 October 11 October 12 October 13 October 14 October 15 | 16 16 16 16 16 16 16 16 16 16 16 16 16 16 16 | 19 19 19 19 19 19 19 19 19 19 19 19 19 19 19 | October 16 October 17 October 18 October 19 October 20 October 21 October 22 October 23 October 24 October 25 October 26 October 27 October 28 October 29 October 30 October 31 | 16 16 16 16 17 17 17 17 17 17 17 17 17 17 17 17 | 19 19 19 19 19 19 19 19 19 19 19 20 20 20 20 20 |

November

| Day | ON delay (minutes) | OFF delay (minutes) | Day | ON delay (minutes) | OFF delay (minutes) |
| --- | --- | --- | --- | --- | --- |
| November 1 November 2 November 3 November 4 November 5 November 6 November 7 November 8 November 9 November 10 November 11 November 12 November 13 November 14 November 15 | 17 17 17 17 17 17 17 17 17 17 17 18 18 18 18 | 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 | November 16 November 17 November 18 November 19 November 20 November 21 November 22 November 23 November 24 November 25 November 26 November 27 November 28 November 29 November 30 | 18 18 18 18 18 18 18 18 18 18 18 18 18 18 18 | 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 |

December

| Day | ON delay (minutes) | OFF delay (minutes) | Day | ON delay (minutes) | OFF delay (minutes) |
| --- | --- | --- | --- | --- | --- |
| December 1 December 2 December 3 December 4 December 5 December 6 December 7 December 8 December 9 December 10 December 11 December 12 December 13 December 14 December 15 | 18 18 18 19 19 19 19 19 19 19 19 19 19 19 19 | 20 20 20 20 20 20 20 20 20 20 20 20 20 20 20 | December 16 December 17 December 18 December 19 December 20 December 21 December 22 December 23 December 24 December 25 December 26 December 27 December 28 December 29 December 30 December 31 | 19 19 19 19 19 19 19 19 19 19 19 20 20 20 20 20 | 20 20 20 20 20 20 20 20 20 20 20 21 21 21 21 21 |

## Traffic signal dimming

### Application of clause 14.6

### This clause 14.6 applies only to jurisdictions where traffic signals are classified as a market load.  Traffic signals that are dimmed have characteristics of both “controlled unmetered loads” and “uncontrolled unmetered loads”.  

These traffic signals loads are similar to “controlled unmetered loads” as they have specifically defined periods that calculated metering data is based on full load values and dimmed load values.  These loads are also similar to “uncontrolled unmetered loads” as they do not have a constant load and calculated metering data is based on an annual energy consumption for the load.Note:

### The effective date of clause 14.6 is 1 July 2015.

### The methodology for production of calculated metering data for dimmed traffic signals must be applied to periods from the clause 14.6 effective date stated in (b).

### Revision of calculated metering data for periods before the clause 14.6 effective date stated in (b) must be undertaken in accordance with the methodology detailed in clause 14.4.

    - Metering Data calculation

      - The metering coordinator must ensure that the interval metering data for traffic signal unmetered loads, which have been classified as a type 7 metering installation and can be dimmed, are calculated in accordance with the following algorithm:

#### (Half-hourly metering data for full wattage trading interval j for NMI – in watt hours) + (Half-hourly metering data for dimmed wattage trading interval j for NMI – in watt hours).

#### =i=1nk*Device full wattagei*Device count for NMIi*Period full wattage switced onj*Trading interval60

#### +i=1nk*Device dimmed wattagei*Device count for NMIi*Period dimmed wattage onj*Trading interval60

where;

i = device type

j = trading interval

k = proportion of device attributable to that NMI

Trading interval is period in minutes

Device full wattage is determined from the load table.

Device dimmed wattage is determined from the load table.

Device count is determined from the inventory table.

Period full wattage switched on is determined from on/off table.

Period dimmed wattage switched on is determined from on/off table.

    - Inventory Table

      - For each NMI, a separate inventory table is required that identifies each device type which forms part of the NMI load and for each device type lists:

- The device type;

- The form of on/off control – photoelectric cell control or timer control;

- If photoelectric cell control, sunset and sunrise times;

- If timer control, the on/off times for the controlling device;

- If a device is shared with another NMI, the proportion of load that is agreed by affected Registered Participants to be attributable to that NMI (k).  Each k factor will be less than 1.  The sum of the k factors for a shared device across each respective NMI must be equal to 1;

- Number of such devices installed;

- Effective start date – the first day on which that record in the inventory table is to be included in the calculation of metering data for that NMI;

- Effective end date – the last day on which that record in the inventory table is to be included in the calculation of metering data for that NMI; and

- Last change date – the date that record in the inventory table was most recently created or modified.

      - Each device in the inventory table is a unique combination of physical hardware, time control classification and shared portion, for example, if a device is shared with another NMI, the individual portions of the device(s) must be included in the inventory table as a separate device type.

      - Each metering coordinator must develop the initial inventory table for the NMIs for which it is responsible.  The initial inventory table must be agreed by the affected Registered Participants, AEMO and the relevant end-use customer.

      - Each metering coordinator must use reasonable endeavours to update the inventory table, for the NMIs for which it is responsible, on at least a monthly basis for any additions, deletions and modifications to ensure that the accuracy requirements in clause 3.7.7 of Metrology Procedure: Part A are met.  Such additions, deletions or modifications to the inventory table may only be made on a retrospective basis where:

- Agreed by the metering coordinator and the affected Registered Participants; or

- Necessary to comply with clause 7.9.5 of the Rules.

      - The metering coordinator must communicate any material changes to the inventory table to the affected Registered Participants.

      - The metering coordinator must provide the inventory table to relevant Registered Participants when requested.

### On/Off table

#### The form of on/off control may be:

##### Photoelectric cell control; or

##### Timer control.

### Photoelectric cell control

#### If the on/off times for the dimming operation is controlled by a photoelectric cell, then:

##### Dimming on time = sunset time

##### Dimming off time = sunrise time.

#### The metering coordinator must ensure that the appropriate sunset times and sunrise times are obtained from the Australian Government Geoscience website (www.ga.gov.au/geodesy/astro/sunrise.jsp), based on the longitude and latitude of the relevant town as specified in clause 14.3.5(b), and Eastern Standard Time:

#### The metering coordinator must ensure that the period that the load is operated at dimmed wattage during a trading interval and the period that the load is operated at full wattage during a trading interval are calculated as follows:

| Trading interval | Period load is switched on |
| --- | --- |
| For the trading intervals commencing after sunset and finishing prior to sunrise | Period dimmed wattage is switched on = 1 |
| For the trading intervals commencing after sunrise and finishing prior to sunset | Period full wattage is switched on = 1 |
| For the trading interval during which the sunset occurs | Period dimmed wattage switched on=End time of trading interval-Time of sunset30 Period full wattage switched on=1-End time of trading interval-Time of sunset30 |
| For the trading interval during which the sunrise occurs | Period dimmed wattage switched on=Time of sunrise-Start time of trading interval30 Period full wattage switched on=1-Time of sunrise-Start time of trading interval30 |

#### Should testing on the operation of photoelectric cells by an independent party, agreed to by the metering coordinator, affected Registered Participants, AEMO and relevant end-use customer, indicate that the on/off times for a device controlled by a photoelectric cell are influenced materially and consistently by other variables, then AEMO shall revise the metrology procedure accordingly.

### Timer control

#### If the on/off times for the dimming operation is controlled by a timer, then

##### On time = ON time set on timer (dimming operation ON)

##### Off time = OFF time set on timer (dimming operation OFF)

#### The metering coordinator must ensure that the period that the load is switched on during a trading interval is calculated as follows:

| Trading interval | Period load is switched on |
| --- | --- |
| For the trading intervals commencing after on time and finishing prior to off time | Period dimmed wattage is switched on = 1 |
| For the trading intervals commencing after off time and finishing prior to on time | Period full wattage is switched on = 1 |
| For the trading interval during which the on time occurs | Period dimmed wattage switched on=End time of trading interval-On time30 Period full wattage switched on=1-End time of trading interval-On time30 |
| For the trading interval during which the off time occurs | Period dimmed wattage switched on=Off time-Start time of trading interval30 Period full wattage switched on=1-Off time-Start time of trading interval30 |

# Substitution READS for transfer

## Application of clause 15

### The requirements of clause 15.2 and 15.3 apply during a retailer of last resort event or where a participating jurisdiction has requested AEMO to undertake end-use customer transfers requiring metering data substitutions.

### The requirements of clause 15.2 are applicable to all type 4A and 5 metering installations requiring the management of metering data substitutions and forward estimations for end-use customer transfer.

### The requirements of clause 15.3 are applicable to all type 6 metering installations requiring the management of metering data substitutions and forward estimations for end-use customer transfer.

## Type 5 metering installations

### The Metering Data Provider must ensure the correct management of the NMI data stream status within MSATS for the connection points concerned that meets Jurisdictional requirements for settlements and profile preparation.

### The Metering Data Provider must ensure that metering data is provided to the old financially responsible Market Participant up to the transfer date.

### The Metering Data Provider must ensure that metering data is provided to the new financially responsible Market Participant from the transfer date which may include provision of a new forward estimation.

### The Metering Data Provider must ensure forward estimations are undertaken in accordance with clause 3 of Metrology Procedure: Part B.

### The Metering Data Provider may substitute the metering data up to the transfer date in order to facilitate end-use customer end billing.  Notification to the Local Network Service Provider, Local Retailer, the old and new financially responsible Market Participant is via the metering data file, that has a format compliant with the Meter Data File Format Specification, with the following configuration:

#### Utilise a reason code of “27”, with an entry in the free text field of “AEMO directed substitution”, and

#### Utilise a transaction code of N.

### The Metering Data Provider must ensure that all substituted metering data is replaced by actual metering data as per the normal manual meter reading processes.

## Type 6 metering installations

### The Metering Data Provider must ensure the correct management of the NMI data stream status within MSATS for the connection points concerned that meets Jurisdictional requirements for settlements and profile preparation.

### The Metering Data Provider must ensure that metering data is provided to the old financially responsible Market Participant up to the transfer date.

### The Metering Data Provider must provide the necessary type 6 substitution readings labelled with a ‘Final’ quality flag.

### The Metering Data Provider must ensure that metering data is provided to the new financially responsible Market Participant from the transfer date which may include provision of a new forward estimation.

### The Metering Data Provider must ensure forward estimations are undertaken in accordance with clause 4 of Metrology Procedure: Part B.

### The Metering Data Provider must calculate the Final substitution reading value in accordance with clause 4 of Metrology Procedure: Part B.

### The Metering Data Provider must notify the Local Network Service Provider, Local Retailer, the old and new financially responsible Market Participant for the connection point of the data substitution undertaken.  Notification is via the metering data file, that has a format compliant with the Meter Data File Format Specification, with the following configuration:

#### Utilise a reason code of “27”, with an entry in the free text field of “AEMO directed substitution”; and

#### Utilise a transaction code of ‘N’.

### The Metering Data Provider must validate the final substituted readings, recalculate and update the metering data values where:

#### The final substitution is found to be greater than the actual metering data when next obtained; or

#### Following consultation and agreement from the old and new financially responsible Market Participant, the Local Retailer and the Local Network Service Provider for the connection point a new agreed value as per clause 4.3.4 (Type 64) must be provided.